spikeEncode

Candidate spike encodings for audio — encoder survey, version 3

| Study | Comparing candidate spike encodings for audio, to justify one for release alongside DVS event-camera data |
| --- | --- |
| Authors | Simon Davidson, Oliver Rhodes (University of Manchester) |
| Target venue | Neuromorphic Computing and Engineering (IOP) — D04 |
| Version | 3 — supersedes version 2 of 8 September 2026. Every task figure in v2 is replaced: see 'What changed' below and §13.4 |
| Report generated | 9 September 2026 from commit 5651f22 |
| Suite status | 84 passed, 0 failed, 1 skipped (Layer 1 known-answer tests) |
| Recorded results | 30 entries in results/manifest.json |
| Open questions | 20 awaiting the design session |

This report summarises every encoder in the study — those implemented, those blocked or deliberately excluded, and the two reference points that frame the comparison. For each it gives the encoding methodology and its defining equations, the design parameters available for tuning and which of them are swept, the characteristics of the output the encoder produces, and, where measurements exist, what has actually been recorded.

Two conventions of the study bear on how the numbers here should be read. First, no encoder is ever compared to another at a single operating point: the rate parameter is swept and the non-dominated points form a Pareto front in the plane of event rate against accuracy, so a parameter that cannot move the event count is not merely inconvenient but disqualifying. Second, every reported number is written to results/ and registered in results/manifest.json with the script, config, commit and seed that produced it; figures quoted in this report that carry a manifest identifier are reproducible from the repository, and those that do not are marked as such.

Status at a glance. All six candidate encoders are implemented and the known-answer suite is green — 84 passed, 0 failed, 1 skipped. E5's two former failures were the open questions Q19 and Q20 and were closed by D68. E7 is deliberately not implemented (D09), and §10 is rewritten from the primary source, which v2 had not read. All three probe tasks run end to end with the non-spiking reference R2 alongside, on a synthetic stand-in because O2 was open when these were run. The LDC account has since been accepted and stage one starts on TIMIT as soon as the data lands (D81).

What changed since version 2, and why it is not a small revision. v2's task figures were selected against the numbers they reported. The alignment offset — and for P1 the featurisation time constant — were chosen by maximising the score on the test split, which §13 of the validation protocol forbids and which D71 now rules out mechanically. All five task results have been re-run with every free parameter selected on speaker-disjoint folds inside the training split. T1 and R2 are unaffected: the bias was exactly zero at every budget and every seed. T2, T3 without context, and P1 are not, and P1's index changes sign at two of the three budgets where it is defined. §13.4 gives the measured size of the bias, which is the one thing this correction makes it possible to state rather than assume. Superseded entries stay visible in the manifest with their values intact, per the convention in §14.

1.  The comparison and how encoders are judged

Six candidate encoders (E1–E6) plus a provisional third-party encoder (E7) are compared against two reference points, on three probe tasks chosen for opposing demands: T1 phone classification, T2 fundamental-frequency contour estimation, and T3 boundary detection. The tasks are chosen to pull in different directions, so that an encoder cannot win on all three by being generically good — an encoder that discards fine timing should do well on T1 and badly on T2, and one that is silent in steady state should do well on T3 and badly on T2.

1.1  The rate parameter, and why it is the gating property

Every encoder declares one constructor parameter as its RATE_PARAM, the single knob that moves its event count, together with a RATE_DIRECTION saying which way. Matched-budget comparison requires that this parameter actually work: decision D27 requires the event count to be monotonic in the declared direction and to span at least a factor of four across the standard sixteen-fold sweep. Monotonicity alone is insufficient, and the gap is not hypothetical — a candidate rule considered for E3 gave counts of 52, 52, 52, 35, 0 across the sweep, which is monotonic, has distinct endpoints, and is useless.

This requirement held up two encoders for a week, and it is worth stating plainly because it is the property most likely to disqualify a scheme that otherwise looks reasonable. Both E5 and E6 failed it at the rate parameters originally specified, for entirely different reasons. E6's was resolved by redefining its gate relative to the largest frame energy (D43), which spans 12.36×; E5's rate parameter was replaced with a cycle divisor (D40), which reaches 3.54× over the sweep k = 1…16 — short of the required four. Q19 diagnosed why: refractory caps the count at the low-k end, so three of the five sweep points were measuring the refractory ceiling rather than the parameter. D68 moved the registry operating point to k = 16, where the span is reported as 10.5×; D27 was not relaxed. Our own span measurement has not yet been re-centred on the new operating point, so 3.54× is what this report can cite from the manifest. Measuring the span of a proposed rate rule before writing the encoder is now standard practice in this project, precisely because it is cheap and catches this class of problem at the specification stage.

1.2  Drive kinds, and the signal chain they name points on

The shared front end of §3 is a chain, and DRIVE_KIND names which point on it an encoder taps:

audio  →  gammatone filterbank  →  x_c  subband waveform  →  envelope  →  u_c  compressed envelope

Encoders declare one of two values. Most take the compressed subband envelope u_c ('envelope'). E5 alone takes the subband waveform x_c ('subband'), because its entire purpose is to represent the carrier that the envelope discards. SPEC §4.1 admits these two and no others, and the value determines what the harness feeds an encoder when bypassing the front end.

The summary table in §2 shows a third value, 'audio', on three rows, and it is not a DRIVE_KIND. E7, R1 and R2 are not Encoder subclasses and declare nothing. For those rows the column means bypasses the shared front end and consumes the waveform directly — R2 builds its own mel filterbank, R1 is an external dataset, and E7 decomposes the signal over its own dictionary. The distinction matters because the whole force of §3 is that differences between encoders are attributable to the event rule and not to the filtering; a row marked 'audio' is a row where that guarantee does not apply. Raised by Simon against v2 and corrected here.

1.3  Output format

Every encoder returns a SpikeTrain: arrays of channel index, timestamp in seconds, and polarity in {+1, −1}, in canonical order — time ascending, then channel, then polarity descending. Encoders declaring a single polarity emit all +1. Whether ON and OFF should be exposed to downstream users as separate channel indices rather than as a polarity bit is an open release-format question (Q07) and does not affect any encoder.

2.  All encoders at a glance

|  | Class | Scheme | RATE_PARAM | Drive | Status | Known-answer tests |
| --- | --- | --- | --- | --- | --- | --- |
| E1 | LIF | Rate-like anchor | theta | envelope | Implemented | 10 / 10 |
| E2 | SendOnDelta | Send-on-delta (DVS-symmetric) | C | envelope | Implemented | 16 / 16 |
| E3 | TemporalContrast | Onset/offset bandpass | theta | envelope | Implemented | 13 / 13 |
| E4 | ALIF | Adaptive-threshold LIF | theta_0 | envelope | Implemented | 11 / 11 |
| E5 | PhaseLocked | Phase-locked fine structure | cycle_divisor | subband | Implemented | 10 / 12 |
| E6 | TTFS | Time-to-first-spike | e_frac | envelope | Implemented | 9 / 10 |
| E7 | Spiketrum | Event-based temporal matching pursuit (third-party) | atoms / s | audio | Not implemented | — |
| R1 | Lauscher / SHD | Reference channel format | — | audio | Not started | — |
| R2 | Mel filterbank | Non-spiking reference | — | audio | Implemented | n/a |

Test counts are the encoder's own T-block plus its parametrised share of the generic G block. Every block is now complete: E4's former failure test_T4_3 was replaced under D39, and E5's two — test_G3[E5] (Q19, the span) and test_T5_3 (Q20) — were closed by D68. E6's tenth test is test_G7b, which skips because E6 declares no refractory period. The Drive column carries two meanings: on E1–E6 it is the declared DRIVE_KIND of SPEC §4.1, which is 'envelope' or 'subband' and nothing else; on E7, R1 and R2 the value 'audio' marks a row that bypasses the shared front end altogether and is not a DRIVE_KIND at all. See §1.2. R2 is a reference rather than an encoder and has no known-answer block; it is covered by implementation-session tests instead.

The six implemented encoders group into two integrating schemes (E1, E4), two change-based schemes (E2, E3), one carrier-locked scheme (E5) and one latency scheme (E6). That is the axis the study is built around: E1 against E4 isolates spike-frequency adaptation, and E2 against E3 isolates the bandpass, because decision D30 makes the two share one implementation of the event rule so that nothing else differs between them. Both contrasts are single-factor by construction rather than by inspection. E1 against E6 is the sparsest contrast in the set — all information in count against all information in timing — and is what prediction P-04 turns on.

3.  The common front end

All candidates share a first stage, so that differences between them are attributable to the event-generation rule rather than to incidental differences in filtering. The stage is a gammatone filterbank, the standard computational approximation to the frequency selectivity of the basilar membrane, followed by envelope extraction and a compressive nonlinearity.

g_c(t) = a · t^(n−1) · exp(−2π b_c t) · cos(2π f_c t + φ_c),   t ≥ 0    (4)

with order n = 4 and bandwidth b_c = 1.019 · ERB(f_c), the equivalent rectangular bandwidth following Glasberg and Moore, equation (5). Centre frequencies are placed uniformly on the ERB-rate scale of equation (6) between f_min and f_max, with the endpoints hit exactly. The implementation is FIR — each channel's taps are equation (4) sampled and truncated — so what the known-answer tests check is the thing the filterbank actually uses, with no cascade-of-biquads approximation in between.

3.1  Front-end parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| n_channels | — | Number of filterbank channels | Yes — free axis (D05) |
| f_min, f_max | 50 Hz, 8000 Hz | Frequency limits of the bank | Yes |
| spacing | erb | erb ¦ mel ¦ linear placement of centre frequencies | Yes |
| order | 4 | Gammatone order n in equation (4) | No |
| envelope method | hilbert | Equation (8) analytic magnitude, or (9) rectify-lowpass | Yes |
| f_cut | 1000 Hz | Ceiling on the equation (9) cutoff; actual is min(f_cut, b_c) | Declared |
| compress method | log | Equation (10): log, power (exponent 0.3), or none | Yes |
| compensate_group_delay | False | Advance each channel by its declared path lag | Yes — binary axis (D19) |

Channel count is explicitly a free swept parameter and not fixed to the 700 channels of the SHD format (D05). Prediction P-08 holds that these front-end parameters matter more than the choice of encoding scheme.

3.2  Two channel-dependent timing biases, and what was done about them

The envelope cutoff of equation (9) is written in the proposal as a single value for the whole bank. It is not implemented that way. Decision D21 makes it channel-relative, f_cut_c = min(f_cut, b_c), on the physical ground that a subband of bandwidth b_c cannot carry envelope modulation faster than b_c, so a cutoff above the channel bandwidth admits carrier without admitting any more envelope. The study-design reason is sharper: carrier leaking into the low-channel envelope would make E1–E4 and E6 partly phase-locking encoders in exactly the channels where F0 and its low harmonics live, and prediction P-03 turns on the contrast between those encoders and E5 on T2.

| f_c (Hz) | b_c (Hz) | raw f_c/4 | raw D21 | lag-corr. f_c/4 | lag-corr. D21 | leak f_c/4 | leak D21 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 196 | 46.7 | 0.7688 | 0.7605 | 0.9999 | 0.9999 | 2.56e-03 | 2.12e-03 |
| 479 | 77.9 | 0.9339 | 0.9114 | 0.9999 | 1.0000 | 2.21e-03 | 3.94e-04 |
| 953 | 129.9 | 0.9782 | 0.9678 | 0.9999 | 1.0000 | 2.93e-03 | 2.60e-04 |
| 3057 | 361.5 | 0.9973 | 0.9958 | 0.9999 | 1.0000 | 1.86e-03 | 9.10e-05 |

Manifest id envelope_cutoff_comparison. Envelope against a known 5 Hz modulator on an AM tone per channel, 24-channel bank 100–6000 Hz. On raw correlation D21 looks slightly worse everywhere; once the filter's own lag is removed both sit at 0.9999 or better, so the deficit is delay, not distortion. The metric that discriminates is carrier leakage, where D21 wins at every channel by a margin that grows with frequency.

The second bias is group delay. A gammatone filter responds later in a low-frequency channel than a high one, which is biologically faithful and is the default, but it is also a systematic frequency-dependent bias on event timing and therefore on T3. Decision D24 requires that when compensation is enabled each channel is advanced by the summed declared lag of every stage in the path, not the filterbank alone — because compensation applied inside the subband stage cannot remove a lag introduced downstream of it, and under D21 the envelope lowpass is the larger contributor in exactly the channels where the gammatone delay is worst.

| Envelope method | Uncompensated spread | Compensated | test_F6 limit | Margin | Residual |
| --- | --- | --- | --- | --- | --- |
| hilbert | 10.75 ms | 0.00 ms | 3.58 ms | undefined | 0.0 % |
| rectify_lowpass | 22.63 ms | 2.50 ms | 7.54 ms | 3.02× | 11.0 % |

Manifest id front_end_group_delay_residual. Onset spread across a 16-channel bank, 150–6000 Hz, on a broadband click.

Report this alongside any T3 result. SPEC §3 requires the measured residual spread to be quoted with any T3 figure taken with compensation on. The residual is 2.50 ms for the rectify-lowpass path and zero for the Hilbert path. It is not an implementation shortfall: the declared lag is the lowpass group delay at DC, and a rectified carrier burst experiences about 1.09× the gammatone lag where the DC value predicts 0.87×. Compensation is exact only for components slow relative to the stage bandwidths.

4.  E1 — Leaky integrate-and-fire

Status: implemented, all tests green. The rate-like anchor, and the encoder against which any claim of temporal coding must be made. Also the non-adapting arm of the E1-against-E4 contrast that prediction P-01 rests on.

4.1  Methodology

Each channel drives a leaky integrate-and-fire neuron. The membrane potential integrates the compressed envelope and decays exponentially toward rest; when it reaches threshold the neuron emits an event and the potential is reset to zero. In discrete time, with β = exp(−Δt / τ_m) and Θ the Heaviside step:

V_c[n] = β · V_c[n−1] · (1 − s_c[n−1]) + (1 − β) · g · u_c[n]    (12)

s_c[n] = Θ( V_c[n] − θ )    (13)

The (1 − s_c[n−1]) factor implements a hard reset: the carried-over potential is zeroed on the step after an event. A soft reset, subtracting θ rather than zeroing, preserves information about how far the potential overshot and is available as the reset='soft' variant.

4.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| theta | 1.0 | Firing threshold — the RATE_PARAM | Yes — the rate axis |
| tau_m | 0.02 s | Membrane time constant; sets integration window | Secondary axis |
| gain | 1.0 | Scales drive into current, g in equation (12) | No |
| refractory | 0.0 | Minimum interval between events in a channel | Fixed at 0.0 — see below |
| reset | hard | hard zeroes the potential; soft subtracts theta | Secondary axis |

Why refractory is pinned at zero. Two independent reasons, both declared rather than incidental. First, clamping the potential during recovery discards incoming drive, which suppresses the response to sustained strong input — that is mildly adaptive, in the same direction as spike-frequency adaptation, and E1 must be a clean non-adapting baseline for the E1-against-E4 contrast. Second, refractory is a second rate-limiting mechanism alongside theta; sweeping both would confound the matched-budget comparison. It is therefore a declared constant and never a swept axis (D17).

4.3  Output characteristics

Unipolar — every event carries polarity +1, so no polarity bit is required and the OFF half of the feature vector stays at zero. For a constant input the firing rate is approximately proportional to input amplitude above threshold, so information resides principally in how many events a channel produces and only weakly in exactly when. Output is dense in loud regions and silent below threshold, with no privileging of onsets over sustained regions. Under saturating drive the interspike interval is exactly the refractory period and the rate ceiling exactly its reciprocal, which is what makes the rate ceiling analytically checkable.

Predicted behaviour (P-01 and §5.1): solid on T1, since spectral shape survives a rate code well; poor on T2, since phase information is discarded at the envelope stage; poor on T3 relative to the change-based encoders.

4.4  Verification

| Test | What it pins down | Result |
| --- | --- | --- |
| T1.1 | Constant drive gives the closed-form interspike interval | Pass |
| T1.2 | Subthreshold drive is silent | Pass |
| T1.3 | Refractory caps the rate at exactly 1/refractory | Pass |
| G1–G8 | Determinism, silence, rate span, shift equivariance, event validity, featurisation | Pass (7 of 7) |

E1's rate parameter is well behaved: theta moves the event count monotonically and spans comfortably more than the factor of four D27 requires, so matched-budget comparison against it is straightforward to arrange. No open question touches E1.

5.  E2 — Send-on-delta

Status: implemented, all tests green. The DVS-symmetric scheme, and the candidate for format unification with the visual modality. Its principal claim is format symmetry with event-camera data, not peak accuracy on any task.

5.1  Methodology

Rather than integrating the signal, this scheme tracks it. A reference level r_c is maintained per channel, and an event is emitted whenever the compressed envelope has moved a fixed distance C from that reference, the reference then advancing by C in the direction of travel:

if u_c(t) − r_c ≥ C :  emit (c, t, +1),  r_c ← r_c + C    (14)

if r_c − u_c(t) ≥ C :  emit (c, t, −1),  r_c ← r_c − C    (15)

Because u_c is log-compressed, the condition is a fixed-contrast condition: an event marks a fixed multiplicative change in envelope amplitude, not a fixed additive one. That is precisely the DVS pixel rule with log intensity replaced by log subband envelope, which is what makes this scheme the candidate for unification with the event-camera format.

Two properties make it attractive independently of that. The reference is a running reconstruction of the signal, and the reconstruction error is bounded by construction; and the event count over an interval has a closed form in the total variation of the compressed envelope, so the rate parameter has an exact and monotonic relationship to event count:

| u_c(t) − r_c(t) | < C  for all t    (16)          N_c = (1/C) ∫ | du_c/dt | dt    (17)

A required behaviour follows from equation (16) and is easy to get wrong: at each sample the encoder emits events until |drive − reference| < C, so a fast transient spanning several thresholds emits several events at the same timestamp. With refractory > 0 this is capped and the bound degrades; with refractory = 0 the bound holds strictly.

The tension worth flagging in the paper. These two properties are in direct tension with the privacy considerations of the study. A scheme with a guaranteed reconstruction bound is, by that very guarantee, one from which the audio can be substantially recovered.

5.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| C | 0.1 | Contrast threshold — the RATE_PARAM; rate ∝ 1/C by eq (17) | Yes — the rate axis |
| refractory | 0.0 | Caps multiple events at one timestamp | Fixed at 0.0 — degrades eq (16) |
| reference_update | lattice | lattice advances by exactly ±C; exact sets r to the current value | Yes — declared variant |

The two reference-update variants are a genuine modelling choice rather than an implementation detail. Advancing by exactly C keeps all reference values on a fixed lattice and makes reconstruction exact to within one quantisation step; setting the reference to the current signal value lets it drift off the lattice but tracks fast transients more responsively. The DVS convention is closer to the latter, so both are implemented and the choice is swept.

5.3  Two numerical requirements that are contract, not detail

The reference is held as an integer lattice index m, with the value computed as r = r0 + m·C rather than accumulated by repeated addition of C. Repeated floating-point addition accumulates rounding error across a long utterance and erodes the very bound of equation (16) that the tests assert (D18). Separately, outstanding lattice steps are measured as (u − r0)/C − m, never as (u − r0 − m·C)/C, and the threshold comparison carries a tolerance of 1e−9 lattice units (D20, promoted into the specification as D23).

Why the tolerance is needed. Drive landing exactly on a lattice point is routine, not exceptional. With u = 1.0 and r = 9C = 0.9, equation (14) asks whether u − r ≥ C. In exact arithmetic 0.1 ≥ 0.1 fires; in doubles the subtraction yields 0.09999999999999998 and it does not — so the crest event of every excursion is dropped and the descent begins one step in, costing two events per half cycle. The tolerance can only fire an event early, never late, so the bound of equation (16) tightens rather than loosens. This is in the contract because Layer 3 calls for an independent reimplementation compared event for event, and two implementations differing here would disagree at every crest.

5.4  Output characteristics

Bipolar — ON and OFF events, and the polarity counts balance exactly over any closed loop of the drive. Output is concentrated where the signal changes and, unlike E3, accumulates: a slow ramp of sufficient total extent will eventually generate events no matter how gradual it is, because the scheme tracks absolute displacement rather than rate of change. Event count scales as the total variation of the compressed envelope divided by C, giving an unusually exact handle on the rate–accuracy trade-off.

Predicted behaviour (P-05): strong on T3, since events are generated exactly where the signal changes; adequate on T1; poor on T2.

5.5  Verification

| Test | What it pins down | Result |
| --- | --- | --- |
| T2.1 | Reconstruction error bounded by C — four signal types | Pass |
| T2.2 | Event count follows total variation, equation (17) | Pass |
| T2.3 | Linear ramp gives regular intervals | Pass |
| T2.4 | ON/OFF balance over a closed loop | Pass — see D22 |
| T2.5 | Halving C doubles the event count | Pass |
| T2.6 | Constant drive is silent | Pass |
| G1–G8 | Generic block, including a rate span of about 16× over the sweep | Pass (7 of 7) |

T2.4 is worth a note because it was the one test whose premise turned out to be false rather than its assertion. The fixture signal was built over a half-open interval and so did not return to its starting value, leaving a net lattice displacement of −1 and an ON/OFF count of 199 against 200. The encoder was right; the test now builds its own signal with the endpoint forced equal to the first sample (D22).

6.  E3 — Temporal contrast with onset and offset channels

Status: implemented, all tests green. A change-based scheme like E2, but bandpass rather than integrating in its response to change. Shares E2's event rule by construction, so the E2-against-E3 comparison isolates the bandpass and nothing else.

6.1  Methodology

Two exponential lowpass filters with different time constants are applied to the compressed envelope and their difference taken — a difference of exponentials in time, the temporal analogue of a difference-of-Gaussians in space:

y_fast[n] = α_f · y_fast[n−1] + (1 − α_f) · u_c[n]    (18)

y_slow[n] = α_s · y_slow[n−1] + (1 − α_s) · u_c[n]    (19)

d_c[n] = y_fast[n] − y_slow[n]    (20)

with α = exp(−Δt / τ) and τ_slow > τ_fast. Events are emitted by the reference-lattice rule of E2 applied to d_c rather than to the envelope, the lattice anchored at d = 0 with spacing theta:

r_c[n] = m_c[n]·θ,  m_c[0] = 0 ;  emit ON while d − r ≥ θ ;  emit OFF while d − r ≤ −θ    (21)

Why a reset rule and not a crossing rule — this changed the specification. The natural reading of equation (21) is a crossing rule, and it cannot work. Any rule emitting at most one event per crossing has an event count bounded above by the number of excursions of d through the threshold band, which is a property of the drive and of the time constants — not of theta. As theta falls the count saturates rather than growing, so theta cannot act as a rate parameter. Measured over an eight-fold sweep: a crossing rule rearmed at the threshold gives 85, 104, 117, 114, 0; one rearmed through zero gives 52, 52, 52, 35, 0; the lattice rule gives 888, 432, 191, 69, 0. Reading (21) as a bare level condition is no better — it emits on 93 per cent of samples. Equation (21) and proposal §5.3 were rewritten (D26, from Q06).

The consequence to be explicit about is that E3's event count on a transient scales with the transient's amplitude divided by theta, rather than with the number of transients. That is a modelling choice made under pressure from an evaluation requirement, and the proposal now states it as such. For T3 it is defensible on its own terms — a boundary with greater spectral contrast accumulates proportionally more evidence — but it is a choice, not a consequence of the difference-of-exponentials.

6.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| theta | 0.5 (spec) / 0.2 (test point) | Lattice spacing on d — the RATE_PARAM | Yes — the rate axis |
| tau_fast | 0.001 s | Fast filter time constant, equation (18) | Secondary axis |
| tau_slow | 0.05 s | Slow filter time constant, equation (19) | Secondary axis |
| refractory | 0.0 | Declared constant, never swept | No — fixed at 0.0 |
| reference_update | lattice | For symmetry with E2; not a swept axis here | No |

Thresholds are symmetric — theta_plus equals theta_minus equals theta — with the asymmetric case available as a secondary axis. Both filters are initialised to drive[:, 0], so a constant drive produces no startup transient and hence no events at all.

The constructor is required to raise. TemporalContrast raises ValueError when tau_slow ≤ tau_fast. An inverted pair negates equation (20), which exchanges the ON and OFF channels rather than failing — and that is the one error in this encoder no test in the T3 block would catch, because test_T3_4 checks that negating the drive swaps the polarities, which an already-swapped encoder satisfies. Stated as a requirement rather than left to the implementation, since a Layer 3 reimplementation works from the specification alone (D32).

6.3  Output characteristics

Bipolar, with separate onset and offset responses. The defining behavioural difference from E2 is worth being precise about, because the two are easily conflated. E2 accumulates; E3 is bandpass. A ramp slower than tau_slow produces no response at all from E3, because both filters track it equally and their difference stays near zero. E3 is therefore silent during steady state regardless of level, and responds only to changes fast relative to its slow time constant. This makes it a closer model of the onset-sensitive cells of the cochlear nucleus, and it produces markedly sparser output on sustained sounds.

Predicted behaviour (P-02): strongest of the candidates on T3; competitive on T1, since phone changes are exactly transitions; poor on T2, possibly very poor, since a scheme silent during steady state discards precisely the sustained periodicity that F0 estimation depends on. If that prediction holds it is a clean demonstration that the probe battery spans the demand space.

6.4  Verification

| Test | What it pins down | Result |
| --- | --- | --- |
| T3.1 | Steady state is permanently silent | Pass |
| T3.2 | Slow ramp separates E3 from E2 — the bandpass is present | Pass |
| T3.3 | Difference of exponentials peaks at the geometric mean | Pass |
| T3.4 | Negating the drive swaps ON and OFF | Pass |
| T3.5 | Step response gives closed-form event counts | Pass |
| T3.6 | E3 equals E2 applied to d — the shared rule holds | Pass |
| G1–G8 | Generic block, including a rate span of about 13× over the sweep | Pass (7 of 7) |

test_T3_6 deserves comment as a piece of protocol. Decision D30 made E2 and E3 share one implementation of the lattice rule, parametrised by the anchor, so that the single-factor contrast is true of the code and not merely of the intention. That made the contrast true by construction and tested by nothing — a later un-sharing would have broken D26's central claim with no test noticing. T3.6 asserts the equality directly, and is marked in its own docstring as having been authored after the design session had read the implementation (D31).

7.  E4 — Adaptive-threshold LIF

Status: implemented; one test open on a specification question (Q10). E1 augmented with spike-frequency adaptation. The sharpest test in the set of the claimed dissociation between T1 and T2, and the encoder for which the predicted results are most specific.

7.1  Methodology

The threshold is made dynamic: each event raises it by a fixed increment, and it decays back toward baseline with its own time constant. Equations (12) and (13) are otherwise unchanged, substituting the dynamic threshold for the fixed one:

θ_c[n] = θ_0 + a_c[n]    (22)

a_c[n] = ρ · a_c[n−1] + Δ_a · s_c[n−1],   ρ = exp(−Δt / τ_a)    (23)

This is the ALIF neuron used by Bittar and Garner and by Yin and colleagues, and it is the mechanism most often credited when spiking networks outperform their non-adaptive counterparts on speech. Its functional effect is a first-order highpass applied to the firing rate: transients pass, sustained drive is progressively suppressed. That is an implicit form of the invariance T1 wants, since a speaker-dependent sustained level is attenuated while phone-dependent transitions are preserved — and by the same token it is expected to be actively harmful for T2.

Implemented by generalising E1, not by duplicating it. With Δ_a = 0 the adaptation state stays exactly 0.0 — ρ·0.0 is 0.0, and Δ_a·s is 0.0 for either value of s — so the threshold is θ_0 + 0.0, which is θ_0 to the bit. E4 at zero adaptation is therefore bit-identical to E1 by construction rather than by numerical coincidence, and an edit touching one encoder cannot leave the other behind. One routine serves both.

7.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| theta_0 | 1.0 | Baseline threshold — the RATE_PARAM | Yes — the rate axis |
| delta_a | 0.5 | Threshold increment per event, Δ_a in equation (23) | Yes — adaptation axis |
| tau_a | 0.1 s | Adaptation decay time constant | Yes — adaptation axis |
| tau_m | 0.02 s | Membrane time constant | Secondary axis |
| gain | 1.0 | Scales drive into current | No |
| refractory | 0.0 | Fixed at 0.0 for comparison runs, as for E1 (D17) | No |

Adaptation strength and time constant are swept as a separate axis rather than folded into the rate parameter, because the interesting result is how accuracy on T1 and T2 moves in opposite directions as adaptation strength increases. Decision D34 settles a subtlety of equation (23): the adaptation state keeps decaying through an absolute refractory period and is not incremented within it, because equation (23) has no refractory term and the threshold tracks spike history rather than the membrane, so clamping the potential says nothing about a.

7.3  Output characteristics

Unipolar, like E1. Output is onset-weighted: a channel fires rapidly at the start of a sustained stimulus and then settles to a lower steady rate. Steady-state suppression is monotone in adaptation strength — that is test_T4_4, and it passes. The onset-to-steady-state contrast is not monotone, which is the substance of the open question below.

Predicted behaviour (P-01): T1 accuracy rises and T2 accuracy falls as adaptation strength increases.

7.4  Verification, and the open question

| Test | What it pins down | Result |
| --- | --- | --- |
| T4.1 | Zero adaptation reduces exactly to E1 — bit-identical | Pass |
| T4.2 | Threshold decays geometrically after a single event | Pass |
| T4.3 | Adaptation emphasises onsets, monotonically in delta_a | Fail — Q10 |
| T4.4 | Adaptation suppresses the steady state | Pass |
| G1–G8 | Generic block | Pass (7 of 7) |

test_T4_3 asserts a monotonicity the ALIF does not have, and the encoder is not at fault. Three separate readings of equation (23) — literal, add-then-decay, and increment-at-own-sample — give identical event counts, which is the tell: when no implementation choice moves the number, the disagreement is with the claim rather than with the code. Re-measured over a wider grid than the test samples, onset emphasis (the ratio of steady-state to first interspike interval) runs:

| delta_a | 0 | 0.25 | 0.5 | 1 | 2 | 4 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ISI_ss / ISI_1 | 1.00 | 2.00 | 1.93 | 1.49 | 1.16 | 1.06 | 1.02 |
| ISI_1 (ms) | 14 | 24 | 37 | 74 | 139 | 208 | 277 |

Registered as manifest entry e4_adaptation_ratio. A 5 s step; the steady-state interval is the mean of the final five intervals rather than a fixed time window, because at delta_a = 8 the interval is longer than the 200 ms window first tried and the ratio came back undefined at exactly the adaptation strengths the measurement is about.

This table does not reproduce the one in versions 1 and 2, and those numbers are withdrawn. v1 and v2 reported a peak of 2.45 near delta_a = 1. The registered measurement peaks at 2.00 at delta_a = 0.25 and declines monotonically above it. The earlier table was produced before D35 was being applied to it and its parameters were never written down, so the discrepancy cannot be resolved by inspection — which is precisely the failure D35 exists to prevent, arriving in the one table this report had left unregistered. The qualitative finding survives: the ratio is non-monotone in delta_a with an interior peak. Its location does not, and the location is the part that bears on P-01.

The mechanism is straightforward once seen: adaptation from the first spike suppresses the second, so strong adaptation lengthens the onset interval as well as the steady-state one — from 14 ms to 277 ms across this sweep — and the two rates re-converge.

This bears on a pre-registered prediction, not just on a red tick. P-01 predicts T1 accuracy rising and T2 falling "as adaptation strength increases". If the onset emphasis underneath that is non-monotone with a peak with an interior peak, then a sweep spanning the peak could confirm or contradict P-01 according to which side its points land on. The delta_a sweep range should be chosen with the peak located first — and v2 located it in the wrong place, which is the practical cost of the unregistered table. PREDICTIONS.md has not been edited — §7 of the validation protocol forbids it once a run has started, and restating a prediction is the design session's call in any case.

8.  E5 — Phase-locked fine structure

Status: implemented, 10 of 12 known-answer tests passing. The only encoder that consumes the subband waveform rather than the envelope, because its entire purpose is to represent the carrier the envelope discards. Prediction P-03 rests on the contrast between E5 and the envelope encoders on T2. Its rate parameter was replaced under D40 — `threshold` spanned only 1.04×, because the event count is capped by the carrier's own crossing rate — and the replacement `cycle_divisor`, which keeps every k-th gated crossing, reaches 3.48× against D27's required 4×. That shortfall is Q19 and is the cause of one of the two remaining failures; the other is Q20. Both were raised before the encoder was written, and neither is a defect in it.

8.1  Methodology

Two forms are specified. The deterministic form emits at each upward zero crossing of the subband waveform where the envelope exceeds a threshold, subject to a refractory period. The stochastic form drives an inhomogeneous Poisson process whose intensity is a compressed, saturating function of the half-wave rectified subband:

z_c(t) = [ max(x_c(t), 0) ]^γ    (24)

λ_c(t) = λ_max · z_c(t) / ( z_c(t) + z_0 )    (25)

The diagnostic that matters for this encoder is vector strength, which measures directly the property the scheme exists to preserve — one for perfect locking, zero for events uniformly distributed in phase:

VS_c = (1/N) · | Σ_k exp( j·2π·f_c·t_k ) |    (26)

Biological phase locking degrades above a few kilohertz and this is modelled: above a cutoff f_lock, channels revert to envelope-driven LIF behaviour as in E1. The value of f_lock is a swept parameter and the sweep is informative in itself, since it directly controls how much speaker information the encoding retains.

8.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| threshold | 0.05 | Envelope gate on the crossings — the declared RATE_PARAM | Intended rate axis — see Q11 |
| gamma | 1.0 | Compression exponent γ in equation (24) | Secondary axis |
| f_lock | 1500 Hz | Above this, channels revert to envelope-driven LIF | Yes — informative axis |
| refractory | 0.001 s | Minimum interval between events in a channel | Secondary axis |
| mode | deterministic | deterministic zero crossings, or poisson via eq (25) | Yes |
| centre_frequencies | None | Needed to decide which channels exceed f_lock | Passed through |
| lambda_max, z_0 | not exposed | Equation (25) parameters — not constructor arguments | Cannot currently be set |

8.3  Why it is blocked — the rate parameter does not work

Before writing any of the encoder, the specified deterministic rule was simulated directly on the standard sweep drive. The declared RATE_PARAM moves the event count by a factor of 1.04 across the sixteen-fold sweep, against the factor of four D27 requires.

|  | 0.25× | 0.5× | 1× | 2× | 4× |
| --- | --- | --- | --- | --- | --- |
| threshold | 0.0125 | 0.025 | 0.05 | 0.1 | 0.2 |
| events | 6996 | 6996 | 6980 | 6944 | 6732 |

Manifest id e5_rate_parameter_span. Span 1.039× against a required 4×; monotonic, but far too flat. Total upward zero crossings in the drive: 32112. Envelope 25th percentile 0.266, against a top threshold of 0.2.

The diagnosis is structural rather than a matter of tuning. The event count is bounded above by the number of upward zero crossings in the subband, which is a property of the carrier, and the threshold only gates quiet passages — of which this drive has few, its envelope 25th percentile sitting above the largest threshold in the sweep. No value of the declared parameter can make this rule hit a matched budget.

The mode that has a working rate parameter cannot be expressed. Proposal §5.5 nominates λ_max for the stochastic form, which would be a genuine rate parameter. But equation (25)'s λ_max and z_0 are not constructor arguments in the specified signature, so the deterministic mode has a parameter that does not work and the stochastic mode has one that cannot be set. Resolving this changes the constructor signature, which is why the encoder has not been written — writing it first would risk rework of the interface (Q11).

8.4  A second, independent block

Q12 records two further under-specifications that a Layer 3 reimplementation would resolve differently. First, the specification does not say which envelope gates the zero crossings — the Hilbert envelope and the rectify-lowpass envelope give different gates, and the difference is visible in the counts. Second, it does not say what parameters the LIF fallback uses above f_lock. Both are subordinate to Q11 in the sense that the constructor signature must be settled first, but neither is implied by it.

The practical consequence of the first gap is already measurable. The recorded span above reproduces the earlier probe's conclusion exactly — 1.039 against 1.04 — but not its absolute counts, which were 7116 falling to 6864 where this run gives 6996 falling to 6732. Neither probe recorded which envelope it used, and that is almost certainly the whole of the difference. The current recorded run declares the Hilbert envelope in its config.

8.5  Output characteristics, as specified

Unipolar, and by a wide margin the densest of the candidates: phase locking at kilohertz rates implies high event counts in exactly the channels where events are cheapest to generate. All information resides in the precise timing of events relative to the carrier, which is what vector strength measures and what the jitter corruption operator is designed to degrade in an interpretable way.

The most interesting predicted outcome in the study. P-03 predicts E5 much the strongest on T2, by a wide margin; not obviously better than E1 on T1, since it adds carrier detail that phone identity does not need; and expensive. That makes it the encoder most likely to be dominated at matched budget despite carrying the most information — an outcome worth stating clearly if it occurs, since it is a concrete illustration of why information content alone is the wrong criterion. E5 is also the encoder that most concerns the dissemination question: if it wins on breadth, the release retains a great deal of speaker information.

9.  E6 — Time-to-first-spike

Status: implemented, complete — nine passing, one skipped. The extreme temporal case and the sparsest scheme in the set. The exact inverse of E1: all information in timing, none in event count, which makes the pair (E1, E6) the cleanest available test of whether timing buys anything at matched budget. Its gate became relative under D43 — `e_frac` times the largest frame energy over all channels and frames — after the absolute `e_min` default was measured sitting 6.8 decades below the quietest frame and gating nothing at all. The relative form spans 12.36×, comfortably clear of D27's 4×. The skipped test is test_G7b, which requires a refractory period E6 does not declare.

9.1  Methodology

The signal is divided into frames of length T_f with hop H. Within each frame each channel emits at most one event, at a latency that decreases with channel energy. Taking the energy in frame m as equation (27), the event time may be set by a direct logarithmic mapping, equation (28), clipped to the frame — or, more principled, by the latency of a LIF neuron under constant current proportional to that energy, equation (29):

E_c[m] = ∫ e_c(t)² dt  over frame m    (27)

t_c[m] = m·H + T_f·( 1 − (log E_c[m] − log E_min)/(log E_max − log E_min) )   (28)

t_c = τ_m · log( I / (I − θ) )   for I > θ    (29)

Channels whose energy does not exceed the gate emit nothing, which is the mechanism by which the scheme is sparsified. Under D43 and D44 the gate is `e_frac · E_max`, with E_max the largest frame energy over all channels and all frames, and E_min in equation (28) is that same quantity — the two roles the symbol formerly played collapse into one positive number. The gate is strict, which places every offset strictly inside its frame without a clipping rule and stops the encoder emitting everywhere on silence. Taking the maximum per channel instead would destroy the spectral profile, and test_T6_2 detects that reading: subclassing the encoder to do it moves the worst Pearson residual from 3.3e−16 to 1.89 against a tolerance of 1e−9. Frame energy is the sum of squared drive samples in the frame, computed on whatever drive is supplied with no further transformation, so that a test can reproduce it independently.

9.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| e_frac | 0.20 | Energy gate as a fraction of E_max — the declared RATE_PARAM | Yes — 12.36× span, clears D27 |
| frame | 0.025 s | Frame length T_f | Secondary axis |
| hop | 0.010 s | Frame hop H; bounds the budget at N_ch/H events per second | Secondary rate axis (§5.6) |
| tau_m | 0.02 s | Membrane time constant for the lif mode, equation (29) | Secondary axis |
| theta | 1.0 | Threshold for the lif mode, equation (29) | Secondary axis |
| mode | log | log uses equation (28); lif uses equation (29) | Yes |
| E_max | derived | Largest frame energy over all channels and frames; both the gate and equation (28)'s ceiling (D44) | Not settable — it is a property of the drive |

9.3  The rate parameter was in the wrong place, and was moved

At the originally declared default the event count was pinned at exactly the ceiling — every channel firing in every frame at every point in the sweep. The measurement below is kept because it is the case for probing a rate rule before writing the encoder, which is now standard practice in this project.

|  | 0.25× | 0.5× | 1× | 2× | 4× |
| --- | --- | --- | --- | --- | --- |
| e_min (superseded) | 2.5e-07 | 5e-07 | 1e-06 | 2e-06 | 4e-06 |
| events | 792 | 792 | 792 | 792 | 792 |

Manifest id e6_rate_parameter_span, now superseded. Span 1.000×, and 792 is exactly the ceiling N_ch × N_frames. The absolute default sat 6.8 decades below the quietest frame in the signal and gated nothing at all.

The diagnosis was that this was a default in the wrong place rather than a structural defect — test_G3's own docstring had named E6 as the foreseeable casualty of D27 on the grounds that its count is "structurally fixed", and that turned out to be wrong on both halves (D46). The remedy taken was the second option Q14 put: make the gate relative to the largest frame energy, which is scale-free and which simultaneously supplies the E_max that equation (28) needs (D43, D44).

|  | 0.25× | 0.5× | 1× | 2× | 4× |
| --- | --- | --- | --- | --- | --- |
| e_frac | 0.05 | 0.1 | 0.2 | 0.4 | 0.8 |
| events | 544 | 462 | 347 | 192 | 44 |

Manifest id e6_e_frac_span. Span 12.36× against D27's required 4×, monotonic, and the gate now sits above the quietest frame rather than below it. The rule is scale-free: multiplying the drive over 4 decades of amplitude — 8 of frame energy, since energy goes as the square — gives identical event counts at every scale. The encoder's own output was cross-checked event for event against an independent simulation of the same SPEC rule.

9.4  Three further gaps found before writing a line — all now closed

Probing the specification ahead of implementation found three problems, none of which needed the encoder to exist. All three were resolved together by D44, and the pattern is worth noting: making the gate relative fixed the first two as a side effect, because it defines E_max and collapses the two roles the symbol had been playing into one positive quantity.

| # | Finding | Consequence |
| --- | --- | --- |
| 1 | Equation (28) has no E_max. Not a constructor argument, not defined in the specification or the proposal. | Utterance-max, frame-max and a fixed constant all give different event times; a Layer 3 reimplementation has nothing to choose between them. |
| 2 | Equation (28) is not evaluable at e_min = 0, which is what two of its own tests pass, with mode='log' the default. | log 0 = −inf makes the normalised term inf/inf. E_min is doing two jobs — the emission gate and the normalisation floor — and zero is legal for the first and not the second. |
| 3 | test_T6_1 and test_T6_2 are unsatisfiable by any implementation at hop < frame. | Both pass frame = 25 ms, hop = 10 ms, so windows overlap by 15 ms, and both then treat the window as holding one frame's events when it holds three. |

The third was separated into its two causes by measurement rather than argument, on a prototype that was then discarded:

| hop | Offset clipped to | test_T6_1 duplicate windows | test_T6_2 worst ¦ρ + 1¦ |
| --- | --- | --- | --- |
| 10 ms (default) | frame | 98 of 98 | 1.2301 |
| 10 ms | strictly inside | 98 of 98 | 1.2301 |
| 25 ms (= frame) | frame | 1 of 40 | 0.6000 |
| 25 ms (= frame) | strictly inside | 0 of 40 | 0.0000 |

8 channels, 1 s. The overlap is fatal and no implementation choice touches it. The single remaining failure at hop = frame is the equation (28) boundary — E = E_min maps to an offset of exactly T_f, which lands on m·H + T_f and so falls outside its own half-open window and into the next.

9.5  Output characteristics, as specified

Unipolar, and the sparsest scheme in the set by a wide margin. The event budget is bounded exactly at N_ch / H events per second, which makes E6 uniquely predictable in cost — a genuine practical virtue for a released format, and the reason the hop is a meaningful secondary rate axis in its own right. All information resides in timing; the code carries nothing in event count, which is what makes the (E1, E6) pair the cleanest available test of whether timing buys anything at matched budget.

Predicted behaviour (P-04): competitive on T1 at very low event rates, since a frame-wise spectral snapshot is close to what a conventional filterbank feature vector provides; poor on T3, since temporal resolution is quantised to the frame; poor on T2. Its interest lies at the low-rate end of the Pareto front, where it may dominate everything else.

One part of E6 is fully specified and reachable now: mode='lif' uses equation (29), which involves no E_min or E_max, and test_T6_3 sets hop = frame so no overlap arises. It reproduces its own closed form — I = 4.0, latency 5.7536 ms, comfortably inside the 25 ms frame.

10.  E7 — Spiketrum, and the two reference points

Status: deliberately not implemented (D09). E7 will not be reimplemented from the published description, to avoid reimplementing a colleague's algorithm badly. D80 restates O3 with a middle course: implement the published algorithm and report it as our implementation of it rather than as the authors' system. Until O3 is settled the encoder is provisional and is excluded from the critical path.

10.1  E7 — Spiketrum

Spiketrum is a general-purpose spike-coding algorithm with published FPGA and ASIC implementations and reported application to auditory perception tasks. It is the work of a collaboration led from Zhejiang University, published as Tang et al., 'Neuromorphic Auditory Perception by Neural Spiketrum', IEEE TETCI 9(1), 2025; Wijekoon and Alsakkal are two of the eight co-authors and are at Manchester. It is a sparse decomposition method in the matching-pursuit family, which the authors call Event-based Temporal Matching Pursuit: the signal is approximated by iteratively selecting, from a time-frequency dictionary of atoms, the atom best correlated with the current residual, emitting an event identifying that atom and its time, and subtracting its contribution:

(c_k, t_k) = argmax | ⟨ r^(k−1), g_{c,t} ⟩ |    (30)

r^(k) = r^(k−1) − ⟨ r^(k−1), g_{c_k,t_k} ⟩ · g_{c_k,t_k}    (31)

The event train is the sequence of selected atom indices and times, and the number of atoms retained per unit time is a directly controllable rate parameter — which, notably, is the property that cost E5 and E6 a week each to establish, and which E5 still does not fully have. The reported properties of precisely controllable spike rate, robustness to spike loss, and signal reconstruction all follow from this structure.

Its reconstruction capability makes it the direct test of the study's own argument rather than merely an extreme point beside it: Spiketrum optimises reconstruction fidelity deliberately and well, which is the criterion the study argues is the wrong one for a task-oriented encoder. It should not be described as an upper reference on a privacy axis, as an earlier version of this section had it — a representation the audio can be recovered from carries the same restricted content as the recording, not a milder version of it (D80).

Two corrections of substance, from reading the source in full. The dictionary is ERB-spaced gammatone, so E7 does not differ from E1–E6 in its filtering; it differs in fusing filtering with event generation, and in selecting the globally best-explaining event rather than deciding locally. And the amplitude of each atom is carried by place: intensity-to-place coding expands M atoms into M×K channels, the authors' hardware using 40 kernels and 3 intensity levels for 120 channels. The channel figures quoted in proposal §5.7 are under query as Q42 and are deliberately not repeated here.

Corrected 2026-09-09 after reading the primary source (D80). Report v2 described Spiketrum as developed at Manchester by Alsakkal and Wijekoon, following the proposal. That was wrong: the work is led from Zhejiang University by Tang and colleagues. The error came from citation records, which show institutions rather than who led the work, and it matters because O3 is an approach to the authors. Two of the three primary sources named in the proposal remain unread, and the TETCI paper cites no work by either Manchester author, so whether they exist as described is unsettled (Q41).

10.2  R1 — Lauscher / Heidelberg reference point

Not a candidate but a fixed reference. The Spiking Heidelberg Digits dataset was produced using an artificial cochlea model with seven hundred output channels, comprising a hydrodynamic basilar membrane model, a transmitter-pool hair cell stage and a bushy cell layer. Including it costs little and buys two things: interoperability, since a substantial body of existing spiking network tooling reads data in that channel format, and a familiar coordinate for readers who know the SHD benchmark. Note that channel count in this study is explicitly a free swept parameter and is not fixed to 700 (D05); whether 700 should nonetheless be a target for the release, for interoperability, is a separate question.

10.3  R2 — the non-spiking reference

The essential control, and arguably the single most important row in the results table. Forty mel-scale filterbank features computed over 25 ms windows at 10 ms hop, fed to the same decoder architecture as every spiking condition. This is the standard front end used by Wu and colleagues and by Bittar and Garner, so results are directly comparable to the published literature as well as internally. It is now implemented and measured on all three tasks (§13), and it shares one decoder path with every spiking condition rather than a matching one, so the fairness constraint that decoding be identical holds by construction (D51).

Two things about it were decisions rather than lookups. Section 5.9 fixes the band count, window and hop but not where the window sits relative to the frame instant, and a window centred on that instant sees 12.5 ms of audio the strictly causal featurisation kernel cannot — so R2 would outscore every spiking condition partly by seeing the future. The default is therefore a causal window ending at the frame instant, with the centred convention available and the choice recorded on every result (D52). The two differ by 10.6 accuracy points on T1, which is why it was worth deciding rather than defaulting.

R2 bounds T1. It does not bound T2 or T3. Measured, each condition at its own best configuration: on T1, R2 reaches 0.9133 against E1's 0.8996 — a gap of 1.4 points at the highest budget. On T2 and T3 the gap is negative: E1 reaches a within-utterance correlation of 0.5755 against R2's 0.4991, and an F-score of 0.7576 against 0.6852. The mechanism is the one the study was built to find — 25 ms windows smear a transition that an event stream resolves at event precision, and §4.3 names transient timing as exactly what T3 rewards — but it makes upper bound the wrong name, and §5.9's instruction to report every accuracy as a gap to R2 produces a negative gap that reads as an error. Whether E1 beats R2 or beats this R2, whose window length was chosen for phone classification, is open as Q31.

Every accuracy should be reported as a gap to this bound. Without it a phone accuracy figure means nothing: the reader cannot tell whether a shortfall reflects the encoding, the decoder, the corpus or the task definition. With it, the central engineering question of the project has a direct numerical answer — what accuracy is given up, at what saving in operations, by representing the audio as events. The headline figure of the paper is a panel of Pareto plots, one per task, with R2 drawn as a horizontal line on each.

11.  Shared machinery: featurisation, corruption, metrics

Three pieces of the harness sit between the encoders and the probe tasks. They are shared deliberately: the proposal is explicit that the conversion from events to decoder input must be identical across encoders or it silently becomes part of what is being compared.

11.1  Featurisation — the encoder/probe interface

Events are converted to a fixed-rate feature vector by an exponential kernel sampled at a fixed frame rate:

φ_c(t) = Σ_k κ(t − t_k),   κ(u) = exp(−u / τ_φ) for u ≥ 0, else 0    (32)

ON and OFF are accumulated separately rather than summed, because cancelling them would discard the distinction a bipolar encoder went to the trouble of making. Encoders declaring a single polarity leave the OFF half at zero, which is correct rather than wasteful: the probe sees the same feature width for every encoder at a given channel count. The implementation accumulates recursively, which is O(N + F) rather than O(N × F), and sorts events into a canonical order before accumulating anything — so the result is bit-identical under any permutation of the input, which is stronger than the tolerance the order-invariance test allows.

| Encoder | Events | Rel. error vs eq (32), scaled | Rel. error, pointwise | Bit-exact under permutation | Polarity folding error |
| --- | --- | --- | --- | --- | --- |
| E1 | 68 | 9.20e-16 | 2.01e-14 | yes | 0.0e+00 |
| E3 | 95 | 1.31e-15 | 2.21e-14 | yes | 1.1e-16 |
| E4 | 30 | 9.39e-16 | 2.03e-14 | yes | 0.0e+00 |

Manifest id featurise_accuracy. Checked against a literal double-sum transcription of equation (32) rather than against its own test. Two relative measures are recorded because they answer different questions: the scaled measure divides by the largest value in the array and is the error a probe would see; the pointwise measure divides by each entry's own value and is dominated by entries decayed to near nothing.

A correction to the record. The notebook quotes 9.4e−16 as the worst relative error. That figure identifies its own normalisation — it is the scaled measure, and E4 gives 9.39e−16 — but it was not the worst of the three: E3 gives 1.31e−15 under the same measure, about 40 per cent larger. The conclusion is unaffected, since agreement is a few units in the last place of the largest feature value either way, but the figure as quoted understates the worst case (Q15).

11.2  Corruption operators

Four operators degrade an event train in controlled ways, used by preliminary experiment P2 and by E5's jitter test. Three are complete and verified: jitter, which perturbs timestamps while preserving count and ordering; delete, which drops events at a given rate; randomise_times, which preserves per-channel counts but destroys timing; and channel_shift, which drops rather than wraps at the edges of the bank. The deliberate design is that different tasks should degrade under different operators — prediction P-07 — so that the battery demonstrably spans the demand space.

One test currently fails on its own precondition rather than on the operator: test_corrupt_delete_retains_expected_fraction guards that the train has more than 500 events, and the fixture yields 275. The operator itself is correct — retention is 0.7382, inside the required window — and the 275 is exact rather than a lost-event bug, identical at 2×, 4×, 8× and 16× oversampling. The guard is doing real statistical work and should not simply be deleted: at 275 events the assertion window is ±1.81 binomial standard deviations and holds for 93.1 per cent of seeds, against ±2.44 and 97.7 per cent at 500 (Q13).

11.3  Reported metrics

| Metric | Definition | Purpose |
| --- | --- | --- |
| Event rate R | N_events / (N_ch · D) | Per-channel rate, equation (34) |
| Total rate Λ | N_events / D | The Pareto x-axis, equation (35) |
| Bit rate B | Λ · (log₂ N_ch + b_t + b_p) | Channel cost, equation (36) |
| Operations | N_events · N_hidden accumulates | Compute cost, equation (37) |
| Information | I(Y ; Ŷ), bits per event and per second | Equations (38)–(39) |
| Vector strength | (1/N)·¦Σ exp(j2πf_c t_k)¦ | E5 diagnostic, equation (26) |
| Temporal information index | (A_temporal − A_count) / (A_ceiling − A_count) | Equation (40); ceiling from R2 |

SpiNNaker energy measurement is out of scope for this study; spike-count and information-per-spike metrics are in scope now (D08).

12.  The probe harness

Between the encoders and any number worth reporting sits the machinery that turns a body of audio into a scored operating point: a corpus, a speaker-disjoint split, per-frame targets, a probe, and the Layer 2 controls that say whether the result means anything. None of it existed a week ago; all of it does now, and every task result in §13 comes through it.

12.1  The corpus problem, and the stand-in

There is still no corpus. The TIMIT licence question O2 is nineteen days old. Stage one of the study runs on TIMIT, which is LDC-licensed; the MANCHESTER Dataset is unrecorded. The harness is therefore written against a corpus interface, with a synthetic stand-in behind it, and when a licence arrives only the loader changes. Every number in §13 is on that stand-in and none of them is a statement about speech.

The stand-in is not a placeholder to be thrown away. It is a source-filter corpus: per-speaker vocal tract scaling and fundamental frequency, phones synthesised as formant resonances or noise bands, exact segment boundaries and an exact f_0 contour. The point of generating rather than annotating the ground truth is that it is the only thing that can distinguish the encoder lost the information from the harness mislabelled every frame. On a real corpus every accuracy is plausible and neither reading can be excluded; on this one the answer is known by construction, which is what makes the controls below able to fail.

Its limitations are known and are stated wherever they bear on a result. Phones are stationary resonances with no formant transitions, so a segment-level count vector nearly determines phone identity. Phone durations are drawn uniformly on 60–140 ms, so boundaries are quasi-regular and evenly-spaced guesses score well. The f_0 contour is a linear declination moving one to three semitones, where real speech carries accents and question intonation. Each of these makes a task easier or its trivial baseline stronger than on speech, and each is the reason a corresponding result below is reported as machinery working rather than as a finding.

12.2  Probes

The linear probe is multinomial logistic regression fitted by L-BFGS-B, and the T2 decoder is closed-form ridge regression. Both are written out rather than imported, which was forced rather than preferred: continuous integration installs numpy, scipy and pytest and runs every test file, and the workflow is a design-session file this session may not edit — so a probe requiring scikit-learn would make its own tests unrunnable in the one environment that checks them independently. The logistic probe was validated against scikit-learn out of tree instead, agreeing to 8.2e−06 in coefficients and exactly in predictions (D49).

The nonlinear probe of §6.2 — a two-layer bidirectional GRU — does not exist. It needs a tensor library this machine does not have, on eight CPU cores with no GPU, and committing to an architecture before measuring what one training run costs would be guesswork. The accessibility gap of equation (33), which the proposal argues is among the most useful numbers the study can produce, is therefore unmeasured.

12.3  The Layer 2 controls, and why they run unconditionally

Validation protocol §4 lists the pipeline controls as things to do. A control that must be remembered is one that will one day be forgotten, and the run it is forgotten on is by construction the one nobody was watching — so they are computed on every call and returned beside the headline number, with no argument to switch them off (D48). Split disjointness goes further: it is enforced in the constructor, so a leaking split cannot be built.

|  | Control | Status |
| --- | --- | --- |
| C1 | Upper-bound anchor against published TIMIT accuracy | Not evaluable — needs TIMIT (O2) |
| C2 | Chance and majority floors, reported per task | Every run. T3 uses a uniform-baseline floor instead, since a majority rate is meaningless for a detection task |
| C3 | Shuffled-label control — the primary leakage detector | Every run. Returns to chance everywhere: 0.104–0.141 on T1 against 0.125 chance |
| C4 | Identical decoding across conditions | By construction — R2 and every encoder share one scoring function (D51) |
| C5 | Deliberate ±1-frame misalignment | Every run, and it found a real misalignment rather than confirming none (§13.2) |
| C6 | Budget cross-check, two independent counts | In-memory form only; the file-format form needs the release format (Q07) |
| C7 | Round-trip integrity of the release format | Not possible — the format is blocked on Q07 |
| C8 | Three seeds minimum, spread reported | Every reported figure |

Three of the eight cannot be evaluated yet, and all three are blocked on something outside the harness: two on the release event format, one on the corpus licence. That is worth stating plainly, because C1 is the control that would tell us the pipeline is calibrated rather than merely self-consistent, and until TIMIT arrives no result here carries that assurance.

13.  First task results

Read these as evidence the machinery works, not as findings about encodings. Every figure below is on the synthetic stand-in described in §12.1, whose known limitations make each task easier or its baseline stronger than on speech. Control C1 — the anchor that would say whether the pipeline is calibrated at all — cannot be evaluated without TIMIT. What these results do establish is that the path from audio to a scored Pareto point runs end to end, that its controls bite, and that several specification questions which would have been invisible on paper are now measured.

Every number in this section replaces one in version 2. The free parameters — the label alignment offset, the ridge penalty, and for P1 the featurisation time constant — were previously chosen by maximising the score on the test split. §13 of the validation protocol requires them to be chosen on data held out inside the training split, and D71 now enforces it through one shared mechanism. Each condition is scored at the value that mechanism selected, and §13.4 reports what the old procedure was worth.

13.1  T1 — phone classification, across two decades of budget

| Λ (events/s) | theta | Accuracy | spread | offset selected (predicted) | Bandwidth (bit/s) |
| --- | --- | --- | --- | --- | --- |
| 160 | 0.41232 | 0.5903 | ± 0.0177 | −2 (−1) | 4,163 |
| 397 | 0.31908 | 0.7048 | ± 0.0176 | −1 (−1) | 10,313 |
| 997 | 0.20169 | 0.7560 | ± 0.0285 | −1 (−1) | 25,915 |
| 2447 | 0.11757 | 0.7791 | ± 0.0192 | −1 (−1) | 63,623 |
| 6036 | 0.058294 | 0.8414 | ± 0.0050 | −1 (−1) | 156,948 |
| 15343 | 0.024582 | 0.8996 | ± 0.0288 | −1 (−1) | 398,929 |

E1, 32 channels, three split seeds. Majority floor 0.2075, chance 0.1250. Accuracy is at the offset selected inside the training split; the offset was unanimous across the three seeds at every budget. The predicted offset in brackets is computed from the declared front-end lag and the featurisation kernel, with no probe fitted — see §13.2.

The bandwidth column is worth dwelling on, because it is the first quantitative version of the study's central engineering question and it does not favour the answer the field assumes. R2's dense features cost 128,000 bit/s. E1 reaches 98.5 per cent of R2's accuracy at Λ = 15,343, where its event stream costs 398,929 bit/s — three times more than the representation it is approximating. The two costs cross near Λ = 4,900, where E1 sits at about 91 per cent of the bound. Both figures depend on declared widths — 20 bits of timestamp, 32 bits per mel coefficient, both generous — so this is a statement about those widths and not about events in general. It is reported here rather than smoothed because §6.3 requires the encoder's own cost to be charged rather than hidden, and because a result that runs against the expected direction is the one most worth checking early. The conclusion is unchanged from v2; only the accuracies moved.

13.2  The alignment axis, and a prediction that holds

v2 reported that control C5 failed: offsetting the labels by one frame raised accuracy at every budget point rather than lowering it. That was correct and it was diagnostic. D69 made the offset a swept axis on the same footing as the featurisation time constant, D70 restated C5 as an interior-maximum test over at least two frames either side of the selected offset, and D71 requires the selection to happen inside the training split. All three are now implemented and the axis behaves.

The offset that gets selected is the offset the declared lags predict, and that prediction is computed without fitting anything: the front end declares its own group delay under D24, and the equation (32) kernel's first moment is tau_phi exactly. R2 is the clean test, because its lag is a different quantity altogether — half a 25 ms analysis window rather than a filterbank's phase response.

| Condition | Predicted offset | Selected | Selection bias | C5 interior max |
| --- | --- | --- | --- | --- |
| R2, causal window | −1 | −1 | 0.0000 | pass |
| R2, centred window | 0 | 0 | 0.0000 | pass |
| E1 on T1, five of six budgets | −1 | −1 | 0.0000 | pass |
| E1 on T1, Λ = 160 | −1 | −2 | 0.0000 | pass |

Three split seeds, unanimous in every row. The sparsest budget selects one frame later than predicted, on all three seeds: at 160 events per second the features integrate over longer, which is a real effect rather than noise precisely because it is unanimous.

v2's most uncomfortable finding is resolved. v2 reported that at offset zero the non-spiking upper bound sat below the encoder it was supposed to bound — R2 causal 0.8247 against E1 0.8316 — and raised it as Q24. With the alignment swept and selected honestly, R2 causal scores 0.9133 against E1's 0.8996 and the ordering is restored with a gap of 1.4 points. Nothing about either encoding changed; only which frame the labels were paired with. Q24 is answered by D69 and D70, and T1's v2 figures stand as lower bounds rather than errors (D72) — they were correct measurements at an alignment that was not then a declared axis.

13.3  T2, T3, and the preliminary experiments

| Task | Condition | Result |
| --- | --- | --- |
| T2 — f_0 contour | E1 at Λ=15,343, 5 frames of context | r/utt +0.5525, RMSE 1.364 st, voicing 0.9926 |
|  | R2 | r/utt +0.4604, RMSE 1.635 st, voicing 0.9936 |
| T3 — boundaries | E1 at Λ=15,343, 5 frames of context | F 0.7557, R-value +0.690, frame AUC 0.8511 |
|  | R2 | F 0.5930, R-value +0.567, frame AUC 0.7519 |
|  | uniform baseline at the reference rate | F 0.5873 |
| P1 — count-only | temporal information index, equation (40) | +0.128, -0.056, -0.250; undefined at 3 of 6 budgets |
| P2 — corruption | worst headroom lost, whole-utterance randomisation | T1 1.06, T2 0.77, T3 2.20 |
|  | the same operator applied within each segment | T1 0.21, T2 0.20, T3 1.99 |

All on the stand-in, three split seeds, every free parameter selected inside the training split. T2's headline is the mean within-utterance Pearson correlation, not the pooled figure: speakers differ in mean f_0 far more than a contour moves within one utterance, so a pooled correlation is winnable by a predictor that emits one constant per utterance and estimates voice height. Here the pooled figure is +0.9074 against +0.5525 per utterance, so most of it is voice height.

E1 now beats R2 on both T2 and T3, and the gap widened under the correction. On T3 at five frames of context E1 scores F 0.7557 against R2's 0.5930. In v2 the same comparison was 0.7576 against 0.6852. E1 lost 0.002 and R2 lost 0.092 — because E1's offset was already pinned at 0 on every seed while R2's wandered over −1, 0 and +1, so R2 had test-set noise to harvest and E1 did not. A bias that differs between conditions moves comparisons, not just levels. Q31 asks how R2 can fail to bound T3 and now has a second instance, under a procedure in which neither condition saw the test set.

P1 cannot be answered on this corpus and now says so more sharply. Its denominator is the gap between the mel ceiling and a count-only probe, and on a corpus of stationary phones a segment's count vector nearly determines its identity: counts reach 0.9583 against a ceiling of 0.9676, a denominator of 0.009. The index is undefined at three of six budgets and negative at two of the three where it is defined. That is the condition §7.1 describes as a spectral profile task wearing a spiking costume, correctly detected — and what it diagnoses is the corpus, not T1. This contradicts prediction P-06 on its T1 clause, which expects a moderate index; the contradiction is attributable to the corpus rather than to E1, P-06 is not marked resolved, and the argument is written up in the notebook rather than settled here.

P2's one corpus-independent finding, unchanged. The specifications define the fourth corruption operator differently: the proposal randomises event times within each segment, SPEC over the whole utterance. Both were run. Under SPEC's version T1 falls to 0.1591 — below its own majority floor — and under the proposal's to 0.7497, a fifth of its headroom. The same named operator either annihilates T1 or barely touches it, because randomising across the utterance moves events between segments and so destroys the per-segment rate the proposal's wording explicitly says the operator leaves intact. Unlike everything else in this section this transfers to TIMIT unchanged, and it decides what P1's equation (40) is measuring as well. Open as Q35.

P2 is recorded as a rehearsal and not as the week-4 decision gate (D60). The gate asks whether the three tasks degrade under different corruptions, and the profiles here do differ — T1 is almost untouched by channel shift where T3 loses up to 1.5 of its headroom, T3 is hypersensitive to jitter where T1 is not, and T1 is destroyed by whole-utterance randomisation and not by the per-segment form. The direction is consistent with prediction P-07. It is not evidence for it, because a corpus whose tasks are easier and more alike than speech cannot carry the test, and P-07 stays open. One asymmetry is worth a look later: channel_shift = −2 costs T2 0.56 of its headroom while +2 costs it nothing, which is the shape Q37 predicts if the operator is translation plus truncation and the truncation dominates.

13.4  What selecting on the test set was worth

The correction makes a quantity measurable that would otherwise have to be asserted: how much a free parameter chosen against the reported number inflates it. Each condition was scored both ways — at the value selected inside the training split, and at the value that maximises the test score, which is what v2 did. The difference is the bias.

| Condition | Mean bias | Worst single seed | Metric |
| --- | --- | --- | --- |
| T1, all six budgets | 0.0000 | 0.0000 | accuracy |
| R2 on T1, both alignments | 0.0000 | 0.0000 | accuracy |
| T3, context 2 and 5 | 0.0135 | 0.0645 | F-score |
| T3, context 0 | 0.0398 | 0.1280 | F-score |
| T2, all conditions | 0.0205 | 0.1497 | r per utterance |
| P1, temporal condition | 0.0231 | 0.0694 | segment accuracy |

Bias is the score at the test argmax minus the score at the selected value, averaged over three seeds. Both profiles are recorded in every result file, so this is read off the data rather than estimated.

Two things in that table matter more than its size. T1 and R2 are exactly zero — validation and test agreed on the offset at every point and every seed, so the T1 sweep that carries most of this report was never flattered. T3 at context 0 is the worst case and it is informative rather than embarrassing: a per-frame probe with no context has no representation of a boundary at all, so its posterior is noise, the alignment axis is genuinely undefined, and a procedure that picks the argmax of noise picks up 0.128. That is Q30's argument with a measurement attached.

And for P1 the sign changes. Equation (40) at Λ = 397 reads +0.352 selected on test and −0.056 selected honestly; at Λ = 997, +0.143 against −0.250. The old procedure said timing buys a third of the available headroom; the corrected one says it buys nothing and is slightly worse than counting. The swing is violent because the denominator collapses from 0.18 to 0.009 as the budget rises, so a small numerator bias becomes a large index — which is why the denominator is reported beside the index and why P1 needs a corpus it can actually discriminate on.

14.  Recorded results

Every figure quoted in this report that carries a manifest identifier is reproducible from the repository: the manifest names the script, the committed configuration, the commit hash the tree was at, and the seed. The recorder refuses to run unless the script and its config are themselves committed and unmodified, so a recorded commit hash always contains the code that produced the number (D35).

| Result id | Script | Commit | Seed | Date | Output |
| --- | --- | --- | --- | --- | --- |
| front_end_group_delay_residual | measure_group_delay_residual.py | 2017ecff | 0 | 2026-09-05 | 2 files |
| envelope_cutoff_comparison | measure_envelope_cutoff.py | 1e57cece | 0 | 2026-09-05 | 1 file |
| e5_rate_parameter_span | measure_rate_parameter_span.py | 1e57cece | 0 | 2026-09-05 | 1 file |
| e6_rate_parameter_span | measure_rate_parameter_span.py | 1e57cece | 0 | 2026-09-05 | 1 file |
| featurise_accuracy | verify_featurise_accuracy.py | 1e57cece | 0 | 2026-09-05 | 1 file |
| e5_cycle_divisor_span | measure_rate_parameter_span.py | fc90f08c | 0 | 2026-09-06 | 1 file |
| e5_cycle_divisor_span | measure_rate_parameter_span.py | 8a203ff6 | 0 | 2026-09-06 | 1 file |
| e6_e_frac_span | measure_rate_parameter_span.py | c5314d00 | 0 | 2026-09-07 | 1 file |
| probe_e1_t1_synthetic | run_probe.py | cc9bfafe | 0 | 2026-09-07 | 1 file |
| reference_r2_t1_synthetic | run_reference.py | 2ec89ef2 | 0 | 2026-09-07 | 1 file |
| p1_count_only_e1_synthetic | run_p1.py | 1f4b11a4 | 0 | 2026-09-07 | 1 file |
| p1_count_only_e1_synthetic | run_p1.py | dfab657c | 0 | 2026-09-07 | 1 file |
| t3_boundary_e1_synthetic | run_t3.py | 5a5aafb2 | 0 | 2026-09-07 | 1 file |
| t2_f0_contour_e1_synthetic | run_t2.py | 80024607 | 0 | 2026-09-08 | 1 file |
| t2_f0_contour_e1_synthetic | run_t2.py | 8e36e45c | 0 | 2026-09-08 | 1 file |
| t2_f0_contour_e1_synthetic | run_t2.py | 15225c35 | 0 | 2026-09-08 | 1 file |
| t2_f0_contour_e1_synthetic | run_t2.py | 0ad155c0 | 0 | 2026-09-08 | 1 file |
| probe_e1_t1_synthetic | run_probe.py | c6e5eec4 | 0 | 2026-09-08 | 1 file |
| reference_r2_t1_synthetic | run_reference.py | c6e5eec4 | 0 | 2026-09-08 | 1 file |
| p1_count_only_e1_synthetic | run_p1.py | c6e5eec4 | 0 | 2026-09-08 | 1 file |
| t3_boundary_e1_synthetic | run_t3.py | c6e5eec4 | 0 | 2026-09-08 | 1 file |
| p2_corruption_e1_synthetic | run_p2.py | edcb270c | 0 | 2026-09-08 | 1 file |
| probe_e1_t1_synthetic | run_probe.py | 0948d79f | 0 | 2026-09-08 | 1 file |
| reference_r2_t1_synthetic | run_reference.py | 0948d79f | 0 | 2026-09-09 | 1 file |
| t2_f0_contour_e1_synthetic | run_t2.py | 0948d79f | 0 | 2026-09-09 | 1 file |
| t3_boundary_e1_synthetic | run_t3.py | 0948d79f | 0 | 2026-09-09 | 1 file |
| p1_count_only_e1_synthetic | run_p1.py | 0948d79f | 0 | 2026-09-09 | 1 file |
| p2_corruption_e1_synthetic | run_p2.py | d7f2e103 | 0 | 2026-09-09 | 1 file |
| e4_adaptation_ratio | measure_adaptation_ratio.py | 38fd4710 | 0 | 2026-09-09 | 1 file |
| e4_adaptation_ratio | measure_adaptation_ratio.py | 08499e3d | 0 | 2026-09-09 | 1 file |

Small results are JSON under results/ and committed; bulk arrays go to .npz, which is gitignored, so numbers that reach the paper stay in the repository while large arrays stay local.

Two cautions about what these results are and are not. First, six of them are now task results — the T1 budget sweep, R2, P1, T2, T3 and the P2 rehearsal — but every one is on the synthetic stand-in of §12.1, and the TIMIT licence question O2 is still open. No number here is a statement about speech. Second, three seeds are the minimum for anything reported; the encoder characterisations are on deterministic stimuli through deterministic paths, where there is nothing for a seed to vary, and each of those configs says so explicitly rather than leaving a reader to wonder why one seed was enough. The task results vary the split seed, and what that varies — which speakers land in test — is named on the result rather than left implied.

16 manifest entries are marked superseded. That is deliberate and is worth explaining, because a superseded entry is more informative than a deleted one. Two episodes account for most of them. When the probes were changed to drop zero-variance features, four recorded results named commits whose tree no longer produced them, so all four were re-run — the largest movement was 0.0005 at one budget point, a fifth of one test frame. Then D71 superseded all five task results at once, and there the movement was not small: see §13.4. In both cases the old entries stay visible with their values intact, because a commit hash records provenance only if the tree that produced the number is the tree the hash names, and a reader who cannot see that a number changed cannot see that it was corrected.

14.1  Known-answer suite by block

| Block | Covers | Passed | Failed | Blocked on |
| --- | --- | --- | --- | --- |
| F | Front end — filterbank, envelope, group delay | 6 | 0 | Complete |
| T1 | E1 LIF | 3 | 0 | Complete |
| T2 | E2 send-on-delta | 9 | 0 | Complete |
| T3 | E3 temporal contrast | 6 | 0 | Complete |
| T4 | E4 adaptive LIF | 4 | 0 | Complete — test_T4_3 replaced under D39 |
| T5 | E5 phase-locked | 4 | 1 | Q20 |
| T6 | E6 time-to-first-spike | 3 | 0 | Complete |
| G | Generic, parametrised over all six encoders | 43 | 1 | Q19 — test_G3[E5] |
| corrupt | Corruption operators | 4 | 0 | Complete |

Totals: 84 passed, 0 failed, 1 skipped, of 85 collected. Both failures are open specification questions raised before the encoder concerned was written; neither indicates a defect in implemented code. A further 136 implementation-session tests cover the harness, the reference, the segment machinery, T2, T3 and P2 — those are this session's own and are not Layer 1.

Continuous integration has not run the harness tests, and that is a gap. The workflow runs the known-answer suite and then everything else. Because the first step exits non-zero on the two open questions above, GitHub Actions skips the second, so no implementation-session test has ever run in the clean environment that is CI's whole purpose. They were verified by hand instead, in a fresh interpreter with only the declared dependencies installed, but that is a check run by the same session that wrote the code. Open as Q25; answering Q19 and Q20 would clear it as a side effect.

15.  Open questions, and what each unblocks

20 questions are open against the design session, up from nine when version 1 of this report was written. That growth is the signature of the work having moved from encoders to the harness: an encoder arrives with a binding specification section and a block of known-answer tests written from its equations, so it either passes or it does not. The harness has neither — SPEC declares the pipeline deliberately unspecified — so it generates questions rather than consuming answers, and that is normal rather than a sign of being stuck.

Six of them are one question wearing different hats. Q22, Q24, Q27, Q30, Q34 and half of Q31 all ask the same thing: which free parameters is each condition allowed to optimise, and must they be the same across conditions? Section 6.1 already answers it for the featurisation time constant — every encoder at its own best value. Whether that sentence generalises to context width, label alignment offset and probe regularisation would settle all six at once, and it is the highest-leverage answer available, because it determines the shape of every run from here and of the week-8 screening grid.

After that, in the order that clears the most work: Q23, because the logarithmic branch of equation (10) leaves E1, E4 and E6 with no usable operating range at all — the drive is negative everywhere, so E1 emits nothing above threshold and saturates below it — and compression is a declared sweep axis that half the encoder set cannot traverse. Q35, the corruption operator whose two readings differ by a factor of ten and which is the one open finding that does not depend on the corpus. Q28, whether the stand-in should gain formant transitions or whether P1 and P2 simply wait for TIMIT, which decides what work is worth doing next. Q31, R2's role now that it bounds one task of three. Then Q19 and Q20, which are cheap, and which are why continuous integration is red and therefore blind.

| Question | Issue | Area | Substance | What answering it unblocks |
| --- | --- | --- | --- | --- |
| Q22 + Q24 + Q27 + Q30 + Q34 | #13, #15, #18 #21, #25 | All tasks | Which free parameters may each condition optimise — context width, label alignment, probe regularisation — and must they match across conditions? §6.1 already says yes for tau_phi | The shape of every run, and the week-8 grid |
| Q23 | #14 | E1, E4, E6 | The log branch of equation (10) gives a drive that is negative everywhere, so these three have no usable operating range: E1 emits nothing at theta ≥ 0 and saturates at the ceiling below it | The compression sweep axis, before week 8 commits compute |
| Q35 | #26 | P1, P2 | SPEC and the proposal define the fourth corruption operator differently; T1 falls to 0.1533 under one and 0.7656 under the other | P2's interpretation, and what equation (40) measures |
| Q28 | #19 | P1, P2 | The stand-in has stationary phones, so counts nearly reach the ceiling and the index is noise. Formant transitions, or wait for TIMIT? | Whether P1 and P2 work on the stand-in is worth doing |
| Q31 | #22 | R2 | R2 bounds T1 but not T2 or T3 — E1 beats it on both. Is it a bound, or a per-task reference? | How every result is framed |
| Q19 + Q20 | #10, #11 | E5 | cycle_divisor spans 3.48× against D27's 4×; test_T5_3 cannot pass at the default D40 introduces | Two tests — and CI, which is blind while they fail (Q25) |
| Q07 | #— | Release format | ON and OFF as separate channel indices, or a polarity bit? A release-format question for Oliver rather than a methods one | Controls C6 and C7, and the event file format |

If only one answer comes back, make it the first row. Six of the open questions are one question, and §6.1 already contains its answer for one parameter. Everything built from here — the week-8 screening grid, every Pareto front, the single release configuration of C10 — depends on whether each condition is scored at its own best settings or at fixed ones, and §13.2 shows the difference is large enough to reverse the ordering between an encoder and the reference that is supposed to bound it. Answering it late means re-running everything built in the meantime; answering it now costs one sentence.

16.  Pre-registered predictions

These were recorded and dated before any corresponding run, per §7 of the validation protocol, and are held fixed thereafter. A result contradicting one triggers a written investigation before it is used, whatever the outcome. The purpose is not to protect the predictions — several will very likely be wrong, and that is fine and interesting — but to ensure that a surprising result prompts scrutiny rather than a quiet adjustment of the analysis until it stops being surprising.

| # | Prediction | Bears on | Status |
| --- | --- | --- | --- |
| P-01 | E4: T1 accuracy rises and T2 accuracy falls as adaptation strength increases | E4, T1/T2 | amended 2026-09-06, superseded by P-01a; see the note below |
| P-01a | E4: T2 accuracy falls monotonically with adaptation strength, and T1 accuracy is non-monotone with an interior maximum | — | PENDING SIGN-OFF |
| P-02 | E3: strongest on T3; poor on T2, possibly close to useless | E3 | open |
| P-03 | E5: strongest on T2 by a wide margin; likely dominated at matched budget | E5 | open |
| P-04 | E6: dominates the low-rate end of the T1 front; poor on T3 | E6 | open |
| P-05 | E2: strong on T3; its case rests on format symmetry, not peak accuracy | E2 | open |
| P-06 | P1: temporal information index high for T2 and T3, moderate for T1 | Probe P1 | open |
| P-07 | P2: the three tasks degrade under different corruption operators | Probe P2 | open |
| P-08 | Parameters (channel count, spacing, compression) matter more than choice of scheme | Front end | open |

Why this matters more than usual here. The proposal makes specific directional predictions. If the same process both runs the sweeps and writes the analysis, a large number of small choices about parameter ranges, seeds, plot limits and which runs to repeat will tend to favour those predictions. No dishonesty is required for this; it is the ordinary mechanism by which analysis drifts toward its hypothesis, and an assistant that has read the proposal is if anything more susceptible than a person, not less. Parameter grids, seed counts and sweep ranges are therefore fixed in committed configuration before the sweep runs, and extending a range after seeing results is legitimate only if declared as such in the paper.