spikeEncode

Candidate spike encodings for audio — encoder survey

| Study | Comparing candidate spike encodings for audio, to justify one for release alongside DVS event-camera data |
| --- | --- |
| Authors | Simon Davidson, Oliver Rhodes (University of Manchester) |
| Target venue | Neuromorphic Computing and Engineering (IOP) — D04 |
| Report generated | 6 September 2026 from commit 007f0b6 |
| Suite status | 61 passed, 23 failed, 1 skipped (Layer 1 known-answer tests) |
| Recorded results | 5 entries in results/manifest.json |
| Open questions | 9 awaiting the design session |

This report summarises every encoder in the study — those implemented, those specified but blocked, and the two reference points that bound the comparison. For each it gives the encoding methodology and its defining equations, the design parameters available for tuning and which of them are swept, the characteristics of the output the encoder produces, and, where measurements exist, what has actually been recorded.

Two conventions of the study bear on how the numbers here should be read. First, no encoder is ever compared to another at a single operating point: the rate parameter is swept and the non-dominated points form a Pareto front in the plane of event rate against accuracy, so a parameter that cannot move the event count is not merely inconvenient but disqualifying. Second, every reported number is written to results/ and registered in results/manifest.json with the script, config, commit and seed that produced it; figures quoted in this report that carry a manifest identifier are reproducible from the repository, and those that do not are marked as such.

Status at a glance. Four of the six candidate encoders are implemented and pass their full known-answer blocks. E5 and E6 are specified but blocked on open questions against the specification, not on implementation effort. E7 is deliberately not implemented (D09). The 23 failing tests are attributable entirely to those blocks and to four individual open questions; none represents a defect in implemented code.

1.  The comparison and how encoders are judged

Six candidate encoders (E1–E6) plus a provisional third-party encoder (E7) are compared against two reference points, on three probe tasks chosen for opposing demands: T1 phone classification, T2 fundamental-frequency contour estimation, and T3 boundary detection. The tasks are chosen to pull in different directions, so that an encoder cannot win on all three by being generically good — an encoder that discards fine timing should do well on T1 and badly on T2, and one that is silent in steady state should do well on T3 and badly on T2.

1.1  The rate parameter, and why it is the gating property

Every encoder declares one constructor parameter as its RATE_PARAM, the single knob that moves its event count, together with a RATE_DIRECTION saying which way. Matched-budget comparison requires that this parameter actually work: decision D27 requires the event count to be monotonic in the declared direction and to span at least a factor of four across the standard sixteen-fold sweep. Monotonicity alone is insufficient, and the gap is not hypothetical — a candidate rule considered for E3 gave counts of 52, 52, 52, 35, 0 across the sweep, which is monotonic, has distinct endpoints, and is useless.

This single requirement is the reason two encoders are currently blocked, and it is worth stating plainly because it is the property most likely to disqualify a scheme that otherwise looks reasonable. Both E5 and E6 fail it at their declared defaults, for entirely different reasons, and the distinction between those reasons determines whether the encoder is salvageable.

1.2  Drive kinds

Encoders consume one of two things. Most take the compressed subband envelope, written u_c in the equations below. E5 alone takes the subband waveform x_c, because its entire purpose is to represent the carrier that the envelope discards. This is declared per encoder as DRIVE_KIND, and it determines what the test harness feeds the encoder when bypassing the front end.

1.3  Output format

Every encoder returns a SpikeTrain: arrays of channel index, timestamp in seconds, and polarity in {+1, −1}, in canonical order — time ascending, then channel, then polarity descending. Encoders declaring a single polarity emit all +1. Whether ON and OFF should be exposed to downstream users as separate channel indices rather than as a polarity bit is an open release-format question (Q07) and does not affect any encoder.

2.  All encoders at a glance

|  | Class | Scheme | RATE_PARAM | Drive | Status | Known-answer tests |
| --- | --- | --- | --- | --- | --- | --- |
| E1 | LIF | Rate-like anchor | theta | envelope | Implemented | 10 / 10 |
| E2 | SendOnDelta | Send-on-delta (DVS-symmetric) | C | envelope | Implemented | 16 / 16 |
| E3 | TemporalContrast | Onset/offset bandpass | theta | envelope | Implemented | 13 / 13 |
| E4 | ALIF | Adaptive-threshold LIF | theta_0 | envelope | Implemented | 10 / 11 |
| E5 | PhaseLocked | Phase-locked fine structure | threshold | subband | Blocked | 0 / 12 |
| E6 | TTFS | Time-to-first-spike | e_min | envelope | Blocked | 0 / 9 |
| E7 | Spiketrum | Matching pursuit (third-party) | atoms / s | audio | Not implemented | — |
| R1 | Lauscher / SHD | Reference channel format | — | audio | Not started | — |
| R2 | Mel filterbank | Non-spiking upper bound | — | audio | Not started | — |

Test counts are the encoder's own T-block plus its parametrised share of the generic G block. E4's single failure is test_T4_3 (Q10); E5 and E6 fail everything because their encode_from_drive raises NotImplementedError.

The four implemented encoders divide cleanly into two integrating schemes (E1, E4) and two change-based schemes (E2, E3). That is the axis the study is built around: E1 against E4 isolates spike-frequency adaptation, and E2 against E3 isolates the bandpass, because decision D30 makes the two share one implementation of the event rule so that nothing else differs between them. Both contrasts are single-factor by construction rather than by inspection.

3.  The common front end

All candidates except E7 share a first stage, so that differences between them are attributable to the event-generation rule rather than to incidental differences in filtering. The stage is a gammatone filterbank, the standard computational approximation to the frequency selectivity of the basilar membrane, followed by envelope extraction and a compressive nonlinearity.

g_c(t) = a · t^(n−1) · exp(−2π b_c t) · cos(2π f_c t + φ_c),   t ≥ 0    (4)

with order n = 4 and bandwidth b_c = 1.019 · ERB(f_c), the equivalent rectangular bandwidth following Glasberg and Moore, equation (5). Centre frequencies are placed uniformly on the ERB-rate scale of equation (6) between f_min and f_max, with the endpoints hit exactly. The implementation is FIR — each channel's taps are equation (4) sampled and truncated — so what the known-answer tests check is the thing the filterbank actually uses, with no cascade-of-biquads approximation in between.

3.1  Front-end parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| n_channels | — | Number of filterbank channels | Yes — free axis (D05) |
| f_min, f_max | 50 Hz, 8000 Hz | Frequency limits of the bank | Yes |
| spacing | erb | erb ¦ mel ¦ linear placement of centre frequencies | Yes |
| order | 4 | Gammatone order n in equation (4) | No |
| envelope method | hilbert | Equation (8) analytic magnitude, or (9) rectify-lowpass | Yes |
| f_cut | 1000 Hz | Ceiling on the equation (9) cutoff; actual is min(f_cut, b_c) | Declared |
| compress method | log | Equation (10): log, power (exponent 0.3), or none | Yes |
| compensate_group_delay | False | Advance each channel by its declared path lag | Yes — binary axis (D19) |

Channel count is explicitly a free swept parameter and not fixed to the 700 channels of the SHD format (D05). Prediction P-08 holds that these front-end parameters matter more than the choice of encoding scheme.

3.2  Two channel-dependent timing biases, and what was done about them

The envelope cutoff of equation (9) is written in the proposal as a single value for the whole bank. It is not implemented that way. Decision D21 makes it channel-relative, f_cut_c = min(f_cut, b_c), on the physical ground that a subband of bandwidth b_c cannot carry envelope modulation faster than b_c, so a cutoff above the channel bandwidth admits carrier without admitting any more envelope. The study-design reason is sharper: carrier leaking into the low-channel envelope would make E1–E4 and E6 partly phase-locking encoders in exactly the channels where F0 and its low harmonics live, and prediction P-03 turns on the contrast between those encoders and E5 on T2.

| f_c (Hz) | b_c (Hz) | raw f_c/4 | raw D21 | lag-corr. f_c/4 | lag-corr. D21 | leak f_c/4 | leak D21 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 196 | 46.7 | 0.7688 | 0.7605 | 0.9999 | 0.9999 | 2.56e-03 | 2.12e-03 |
| 479 | 77.9 | 0.9339 | 0.9114 | 0.9999 | 1.0000 | 2.21e-03 | 3.94e-04 |
| 953 | 129.9 | 0.9782 | 0.9678 | 0.9999 | 1.0000 | 2.93e-03 | 2.60e-04 |
| 3057 | 361.5 | 0.9973 | 0.9958 | 0.9999 | 1.0000 | 1.86e-03 | 9.10e-05 |

Manifest id envelope_cutoff_comparison. Envelope against a known 5 Hz modulator on an AM tone per channel, 24-channel bank 100–6000 Hz. On raw correlation D21 looks slightly worse everywhere; once the filter's own lag is removed both sit at 0.9999 or better, so the deficit is delay, not distortion. The metric that discriminates is carrier leakage, where D21 wins at every channel by a margin that grows with frequency.

The second bias is group delay. A gammatone filter responds later in a low-frequency channel than a high one, which is biologically faithful and is the default, but it is also a systematic frequency-dependent bias on event timing and therefore on T3. Decision D24 requires that when compensation is enabled each channel is advanced by the summed declared lag of every stage in the path, not the filterbank alone — because compensation applied inside the subband stage cannot remove a lag introduced downstream of it, and under D21 the envelope lowpass is the larger contributor in exactly the channels where the gammatone delay is worst.

| Envelope method | Uncompensated spread | Compensated | test_F6 limit | Margin | Residual |
| --- | --- | --- | --- | --- | --- |
| hilbert | 10.75 ms | 0.00 ms | 3.58 ms | undefined | 0.0 % |
| rectify_lowpass | 22.63 ms | 2.50 ms | 7.54 ms | 3.02× | 11.0 % |

Manifest id front_end_group_delay_residual. Onset spread across a 16-channel bank, 150–6000 Hz, on a broadband click.

Report this alongside any T3 result. SPEC §3 requires the measured residual spread to be quoted with any T3 figure taken with compensation on. The residual is 2.50 ms for the rectify-lowpass path and zero for the Hilbert path. It is not an implementation shortfall: the declared lag is the lowpass group delay at DC, and a rectified carrier burst experiences about 1.09× the gammatone lag where the DC value predicts 0.87×. Compensation is exact only for components slow relative to the stage bandwidths.

4.  E1 — Leaky integrate-and-fire

Status: implemented, all tests green. The rate-like anchor, and the encoder against which any claim of temporal coding must be made. Also the non-adapting arm of the E1-against-E4 contrast that prediction P-01 rests on.

4.1  Methodology

Each channel drives a leaky integrate-and-fire neuron. The membrane potential integrates the compressed envelope and decays exponentially toward rest; when it reaches threshold the neuron emits an event and the potential is reset to zero. In discrete time, with β = exp(−Δt / τ_m) and Θ the Heaviside step:

V_c[n] = β · V_c[n−1] · (1 − s_c[n−1]) + (1 − β) · g · u_c[n]    (12)

s_c[n] = Θ( V_c[n] − θ )    (13)

The (1 − s_c[n−1]) factor implements a hard reset: the carried-over potential is zeroed on the step after an event. A soft reset, subtracting θ rather than zeroing, preserves information about how far the potential overshot and is available as the reset='soft' variant.

4.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| theta | 1.0 | Firing threshold — the RATE_PARAM | Yes — the rate axis |
| tau_m | 0.02 s | Membrane time constant; sets integration window | Secondary axis |
| gain | 1.0 | Scales drive into current, g in equation (12) | No |
| refractory | 0.0 | Minimum interval between events in a channel | Fixed at 0.0 — see below |
| reset | hard | hard zeroes the potential; soft subtracts theta | Secondary axis |

Why refractory is pinned at zero. Two independent reasons, both declared rather than incidental. First, clamping the potential during recovery discards incoming drive, which suppresses the response to sustained strong input — that is mildly adaptive, in the same direction as spike-frequency adaptation, and E1 must be a clean non-adapting baseline for the E1-against-E4 contrast. Second, refractory is a second rate-limiting mechanism alongside theta; sweeping both would confound the matched-budget comparison. It is therefore a declared constant and never a swept axis (D17).

4.3  Output characteristics

Unipolar — every event carries polarity +1, so no polarity bit is required and the OFF half of the feature vector stays at zero. For a constant input the firing rate is approximately proportional to input amplitude above threshold, so information resides principally in how many events a channel produces and only weakly in exactly when. Output is dense in loud regions and silent below threshold, with no privileging of onsets over sustained regions. Under saturating drive the interspike interval is exactly the refractory period and the rate ceiling exactly its reciprocal, which is what makes the rate ceiling analytically checkable.

Predicted behaviour (P-01 and §5.1): solid on T1, since spectral shape survives a rate code well; poor on T2, since phase information is discarded at the envelope stage; poor on T3 relative to the change-based encoders.

4.4  Verification

| Test | What it pins down | Result |
| --- | --- | --- |
| T1.1 | Constant drive gives the closed-form interspike interval | Pass |
| T1.2 | Subthreshold drive is silent | Pass |
| T1.3 | Refractory caps the rate at exactly 1/refractory | Pass |
| G1–G8 | Determinism, silence, rate span, shift equivariance, event validity, featurisation | Pass (7 of 7) |

E1's rate parameter is well behaved: theta moves the event count monotonically and spans comfortably more than the factor of four D27 requires, so matched-budget comparison against it is straightforward to arrange. No open question touches E1.

5.  E2 — Send-on-delta

Status: implemented, all tests green. The DVS-symmetric scheme, and the candidate for format unification with the visual modality. Its principal claim is format symmetry with event-camera data, not peak accuracy on any task.

5.1  Methodology

Rather than integrating the signal, this scheme tracks it. A reference level r_c is maintained per channel, and an event is emitted whenever the compressed envelope has moved a fixed distance C from that reference, the reference then advancing by C in the direction of travel:

if u_c(t) − r_c ≥ C :  emit (c, t, +1),  r_c ← r_c + C    (14)

if r_c − u_c(t) ≥ C :  emit (c, t, −1),  r_c ← r_c − C    (15)

Because u_c is log-compressed, the condition is a fixed-contrast condition: an event marks a fixed multiplicative change in envelope amplitude, not a fixed additive one. That is precisely the DVS pixel rule with log intensity replaced by log subband envelope, which is what makes this scheme the candidate for unification with the event-camera format.

Two properties make it attractive independently of that. The reference is a running reconstruction of the signal, and the reconstruction error is bounded by construction; and the event count over an interval has a closed form in the total variation of the compressed envelope, so the rate parameter has an exact and monotonic relationship to event count:

| u_c(t) − r_c(t) | < C  for all t    (16)          N_c = (1/C) ∫ | du_c/dt | dt    (17)

A required behaviour follows from equation (16) and is easy to get wrong: at each sample the encoder emits events until |drive − reference| < C, so a fast transient spanning several thresholds emits several events at the same timestamp. With refractory > 0 this is capped and the bound degrades; with refractory = 0 the bound holds strictly.

The tension worth flagging in the paper. These two properties are in direct tension with the privacy considerations of the study. A scheme with a guaranteed reconstruction bound is, by that very guarantee, one from which the audio can be substantially recovered.

5.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| C | 0.1 | Contrast threshold — the RATE_PARAM; rate ∝ 1/C by eq (17) | Yes — the rate axis |
| refractory | 0.0 | Caps multiple events at one timestamp | Fixed at 0.0 — degrades eq (16) |
| reference_update | lattice | lattice advances by exactly ±C; exact sets r to the current value | Yes — declared variant |

The two reference-update variants are a genuine modelling choice rather than an implementation detail. Advancing by exactly C keeps all reference values on a fixed lattice and makes reconstruction exact to within one quantisation step; setting the reference to the current signal value lets it drift off the lattice but tracks fast transients more responsively. The DVS convention is closer to the latter, so both are implemented and the choice is swept.

5.3  Two numerical requirements that are contract, not detail

The reference is held as an integer lattice index m, with the value computed as r = r0 + m·C rather than accumulated by repeated addition of C. Repeated floating-point addition accumulates rounding error across a long utterance and erodes the very bound of equation (16) that the tests assert (D18). Separately, outstanding lattice steps are measured as (u − r0)/C − m, never as (u − r0 − m·C)/C, and the threshold comparison carries a tolerance of 1e−9 lattice units (D20, promoted into the specification as D23).

Why the tolerance is needed. Drive landing exactly on a lattice point is routine, not exceptional. With u = 1.0 and r = 9C = 0.9, equation (14) asks whether u − r ≥ C. In exact arithmetic 0.1 ≥ 0.1 fires; in doubles the subtraction yields 0.09999999999999998 and it does not — so the crest event of every excursion is dropped and the descent begins one step in, costing two events per half cycle. The tolerance can only fire an event early, never late, so the bound of equation (16) tightens rather than loosens. This is in the contract because Layer 3 calls for an independent reimplementation compared event for event, and two implementations differing here would disagree at every crest.

5.4  Output characteristics

Bipolar — ON and OFF events, and the polarity counts balance exactly over any closed loop of the drive. Output is concentrated where the signal changes and, unlike E3, accumulates: a slow ramp of sufficient total extent will eventually generate events no matter how gradual it is, because the scheme tracks absolute displacement rather than rate of change. Event count scales as the total variation of the compressed envelope divided by C, giving an unusually exact handle on the rate–accuracy trade-off.

Predicted behaviour (P-05): strong on T3, since events are generated exactly where the signal changes; adequate on T1; poor on T2.

5.5  Verification

| Test | What it pins down | Result |
| --- | --- | --- |
| T2.1 | Reconstruction error bounded by C — four signal types | Pass |
| T2.2 | Event count follows total variation, equation (17) | Pass |
| T2.3 | Linear ramp gives regular intervals | Pass |
| T2.4 | ON/OFF balance over a closed loop | Pass — see D22 |
| T2.5 | Halving C doubles the event count | Pass |
| T2.6 | Constant drive is silent | Pass |
| G1–G8 | Generic block, including a rate span of about 16× over the sweep | Pass (7 of 7) |

T2.4 is worth a note because it was the one test whose premise turned out to be false rather than its assertion. The fixture signal was built over a half-open interval and so did not return to its starting value, leaving a net lattice displacement of −1 and an ON/OFF count of 199 against 200. The encoder was right; the test now builds its own signal with the endpoint forced equal to the first sample (D22).

6.  E3 — Temporal contrast with onset and offset channels

Status: implemented, all tests green. A change-based scheme like E2, but bandpass rather than integrating in its response to change. Shares E2's event rule by construction, so the E2-against-E3 comparison isolates the bandpass and nothing else.

6.1  Methodology

Two exponential lowpass filters with different time constants are applied to the compressed envelope and their difference taken — a difference of exponentials in time, the temporal analogue of a difference-of-Gaussians in space:

y_fast[n] = α_f · y_fast[n−1] + (1 − α_f) · u_c[n]    (18)

y_slow[n] = α_s · y_slow[n−1] + (1 − α_s) · u_c[n]    (19)

d_c[n] = y_fast[n] − y_slow[n]    (20)

with α = exp(−Δt / τ) and τ_slow > τ_fast. Events are emitted by the reference-lattice rule of E2 applied to d_c rather than to the envelope, the lattice anchored at d = 0 with spacing theta:

r_c[n] = m_c[n]·θ,  m_c[0] = 0 ;  emit ON while d − r ≥ θ ;  emit OFF while d − r ≤ −θ    (21)

Why a reset rule and not a crossing rule — this changed the specification. The natural reading of equation (21) is a crossing rule, and it cannot work. Any rule emitting at most one event per crossing has an event count bounded above by the number of excursions of d through the threshold band, which is a property of the drive and of the time constants — not of theta. As theta falls the count saturates rather than growing, so theta cannot act as a rate parameter. Measured over an eight-fold sweep: a crossing rule rearmed at the threshold gives 85, 104, 117, 114, 0; one rearmed through zero gives 52, 52, 52, 35, 0; the lattice rule gives 888, 432, 191, 69, 0. Reading (21) as a bare level condition is no better — it emits on 93 per cent of samples. Equation (21) and proposal §5.3 were rewritten (D26, from Q06).

The consequence to be explicit about is that E3's event count on a transient scales with the transient's amplitude divided by theta, rather than with the number of transients. That is a modelling choice made under pressure from an evaluation requirement, and the proposal now states it as such. For T3 it is defensible on its own terms — a boundary with greater spectral contrast accumulates proportionally more evidence — but it is a choice, not a consequence of the difference-of-exponentials.

6.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| theta | 0.5 (spec) / 0.2 (test point) | Lattice spacing on d — the RATE_PARAM | Yes — the rate axis |
| tau_fast | 0.001 s | Fast filter time constant, equation (18) | Secondary axis |
| tau_slow | 0.05 s | Slow filter time constant, equation (19) | Secondary axis |
| refractory | 0.0 | Declared constant, never swept | No — fixed at 0.0 |
| reference_update | lattice | For symmetry with E2; not a swept axis here | No |

Thresholds are symmetric — theta_plus equals theta_minus equals theta — with the asymmetric case available as a secondary axis. Both filters are initialised to drive[:, 0], so a constant drive produces no startup transient and hence no events at all.

The constructor is required to raise. TemporalContrast raises ValueError when tau_slow ≤ tau_fast. An inverted pair negates equation (20), which exchanges the ON and OFF channels rather than failing — and that is the one error in this encoder no test in the T3 block would catch, because test_T3_4 checks that negating the drive swaps the polarities, which an already-swapped encoder satisfies. Stated as a requirement rather than left to the implementation, since a Layer 3 reimplementation works from the specification alone (D32).

6.3  Output characteristics

Bipolar, with separate onset and offset responses. The defining behavioural difference from E2 is worth being precise about, because the two are easily conflated. E2 accumulates; E3 is bandpass. A ramp slower than tau_slow produces no response at all from E3, because both filters track it equally and their difference stays near zero. E3 is therefore silent during steady state regardless of level, and responds only to changes fast relative to its slow time constant. This makes it a closer model of the onset-sensitive cells of the cochlear nucleus, and it produces markedly sparser output on sustained sounds.

Predicted behaviour (P-02): strongest of the candidates on T3; competitive on T1, since phone changes are exactly transitions; poor on T2, possibly very poor, since a scheme silent during steady state discards precisely the sustained periodicity that F0 estimation depends on. If that prediction holds it is a clean demonstration that the probe battery spans the demand space.

6.4  Verification

| Test | What it pins down | Result |
| --- | --- | --- |
| T3.1 | Steady state is permanently silent | Pass |
| T3.2 | Slow ramp separates E3 from E2 — the bandpass is present | Pass |
| T3.3 | Difference of exponentials peaks at the geometric mean | Pass |
| T3.4 | Negating the drive swaps ON and OFF | Pass |
| T3.5 | Step response gives closed-form event counts | Pass |
| T3.6 | E3 equals E2 applied to d — the shared rule holds | Pass |
| G1–G8 | Generic block, including a rate span of about 13× over the sweep | Pass (7 of 7) |

test_T3_6 deserves comment as a piece of protocol. Decision D30 made E2 and E3 share one implementation of the lattice rule, parametrised by the anchor, so that the single-factor contrast is true of the code and not merely of the intention. That made the contrast true by construction and tested by nothing — a later un-sharing would have broken D26's central claim with no test noticing. T3.6 asserts the equality directly, and is marked in its own docstring as having been authored after the design session had read the implementation (D31).

7.  E4 — Adaptive-threshold LIF

Status: implemented; one test open on a specification question (Q10). E1 augmented with spike-frequency adaptation. The sharpest test in the set of the claimed dissociation between T1 and T2, and the encoder for which the predicted results are most specific.

7.1  Methodology

The threshold is made dynamic: each event raises it by a fixed increment, and it decays back toward baseline with its own time constant. Equations (12) and (13) are otherwise unchanged, substituting the dynamic threshold for the fixed one:

θ_c[n] = θ_0 + a_c[n]    (22)

a_c[n] = ρ · a_c[n−1] + Δ_a · s_c[n−1],   ρ = exp(−Δt / τ_a)    (23)

This is the ALIF neuron used by Bittar and Garner and by Yin and colleagues, and it is the mechanism most often credited when spiking networks outperform their non-adaptive counterparts on speech. Its functional effect is a first-order highpass applied to the firing rate: transients pass, sustained drive is progressively suppressed. That is an implicit form of the invariance T1 wants, since a speaker-dependent sustained level is attenuated while phone-dependent transitions are preserved — and by the same token it is expected to be actively harmful for T2.

Implemented by generalising E1, not by duplicating it. With Δ_a = 0 the adaptation state stays exactly 0.0 — ρ·0.0 is 0.0, and Δ_a·s is 0.0 for either value of s — so the threshold is θ_0 + 0.0, which is θ_0 to the bit. E4 at zero adaptation is therefore bit-identical to E1 by construction rather than by numerical coincidence, and an edit touching one encoder cannot leave the other behind. One routine serves both.

7.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| theta_0 | 1.0 | Baseline threshold — the RATE_PARAM | Yes — the rate axis |
| delta_a | 0.5 | Threshold increment per event, Δ_a in equation (23) | Yes — adaptation axis |
| tau_a | 0.1 s | Adaptation decay time constant | Yes — adaptation axis |
| tau_m | 0.02 s | Membrane time constant | Secondary axis |
| gain | 1.0 | Scales drive into current | No |
| refractory | 0.0 | Fixed at 0.0 for comparison runs, as for E1 (D17) | No |

Adaptation strength and time constant are swept as a separate axis rather than folded into the rate parameter, because the interesting result is how accuracy on T1 and T2 moves in opposite directions as adaptation strength increases. Decision D34 settles a subtlety of equation (23): the adaptation state keeps decaying through an absolute refractory period and is not incremented within it, because equation (23) has no refractory term and the threshold tracks spike history rather than the membrane, so clamping the potential says nothing about a.

7.3  Output characteristics

Unipolar, like E1. Output is onset-weighted: a channel fires rapidly at the start of a sustained stimulus and then settles to a lower steady rate. Steady-state suppression is monotone in adaptation strength — that is test_T4_4, and it passes. The onset-to-steady-state contrast is not monotone, which is the substance of the open question below.

Predicted behaviour (P-01): T1 accuracy rises and T2 accuracy falls as adaptation strength increases.

7.4  Verification, and the open question

| Test | What it pins down | Result |
| --- | --- | --- |
| T4.1 | Zero adaptation reduces exactly to E1 — bit-identical | Pass |
| T4.2 | Threshold decays geometrically after a single event | Pass |
| T4.3 | Adaptation emphasises onsets, monotonically in delta_a | Fail — Q10 |
| T4.4 | Adaptation suppresses the steady state | Pass |
| G1–G8 | Generic block | Pass (7 of 7) |

test_T4_3 asserts a monotonicity the ALIF does not have, and the encoder is not at fault. Three separate readings of equation (23) — literal, add-then-decay, and increment-at-own-sample — give identical event counts, which is the tell: when no implementation choice moves the number, the disagreement is with the claim rather than with the code. Re-measured over a wider grid than the test samples, onset emphasis (the ratio of steady-state to first interspike interval) runs:

| delta_a | 0 | 0.25 | 0.5 | 1 | 2 | 4 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ISI_ss / ISI_1 | 1.00 | 2.11 | 2.38 | 2.45 | 2.14 | 1.51 | 1.16 |

Measured on 5 s with 200 ms windows so the counts are adequate and the steady state genuinely reached. Not yet registered in the manifest.

It peaks near delta_a ≈ 1 and decays either side, and the test's two adapting points, 0.5 and 2.0, straddle the peak. The mechanism is straightforward once seen: adaptation from the first spike suppresses the second, so strong adaptation lengthens the onset interval as well as the steady-state one — from 8 ms to 139 ms — and the two rates re-converge.

This bears on a pre-registered prediction, not just on a red tick. P-01 predicts T1 accuracy rising and T2 falling "as adaptation strength increases". If the onset emphasis underneath that is non-monotone with a peak near delta_a ≈ 1, then a sweep spanning the peak could confirm or contradict P-01 according to which side its points land on. The delta_a sweep range should be chosen with the peak located first. PREDICTIONS.md has not been edited — §7 of the validation protocol forbids it once a run has started, and restating a prediction is the design session's call in any case.

8.  E5 — Phase-locked fine structure

Status: specified, not implemented. Blocked on Q11 and Q12. The only encoder that consumes the subband waveform rather than the envelope, because its entire purpose is to represent the carrier the envelope discards. Prediction P-03 rests on the contrast between E5 and the envelope encoders on T2.

8.1  Methodology

Two forms are specified. The deterministic form emits at each upward zero crossing of the subband waveform where the envelope exceeds a threshold, subject to a refractory period. The stochastic form drives an inhomogeneous Poisson process whose intensity is a compressed, saturating function of the half-wave rectified subband:

z_c(t) = [ max(x_c(t), 0) ]^γ    (24)

λ_c(t) = λ_max · z_c(t) / ( z_c(t) + z_0 )    (25)

The diagnostic that matters for this encoder is vector strength, which measures directly the property the scheme exists to preserve — one for perfect locking, zero for events uniformly distributed in phase:

VS_c = (1/N) · | Σ_k exp( j·2π·f_c·t_k ) |    (26)

Biological phase locking degrades above a few kilohertz and this is modelled: above a cutoff f_lock, channels revert to envelope-driven LIF behaviour as in E1. The value of f_lock is a swept parameter and the sweep is informative in itself, since it directly controls how much speaker information the encoding retains.

8.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| threshold | 0.05 | Envelope gate on the crossings — the declared RATE_PARAM | Intended rate axis — see Q11 |
| gamma | 1.0 | Compression exponent γ in equation (24) | Secondary axis |
| f_lock | 1500 Hz | Above this, channels revert to envelope-driven LIF | Yes — informative axis |
| refractory | 0.001 s | Minimum interval between events in a channel | Secondary axis |
| mode | deterministic | deterministic zero crossings, or poisson via eq (25) | Yes |
| centre_frequencies | None | Needed to decide which channels exceed f_lock | Passed through |
| lambda_max, z_0 | not exposed | Equation (25) parameters — not constructor arguments | Cannot currently be set |

8.3  Why it is blocked — the rate parameter does not work

Before writing any of the encoder, the specified deterministic rule was simulated directly on the standard sweep drive. The declared RATE_PARAM moves the event count by a factor of 1.04 across the sixteen-fold sweep, against the factor of four D27 requires.

|  | 0.25× | 0.5× | 1× | 2× | 4× |
| --- | --- | --- | --- | --- | --- |
| threshold | 0.0125 | 0.025 | 0.05 | 0.1 | 0.2 |
| events | 6996 | 6996 | 6980 | 6944 | 6732 |

Manifest id e5_rate_parameter_span. Span 1.039× against a required 4×; monotonic, but far too flat. Total upward zero crossings in the drive: 32112. Envelope 25th percentile 0.266, against a top threshold of 0.2.

The diagnosis is structural rather than a matter of tuning. The event count is bounded above by the number of upward zero crossings in the subband, which is a property of the carrier, and the threshold only gates quiet passages — of which this drive has few, its envelope 25th percentile sitting above the largest threshold in the sweep. No value of the declared parameter can make this rule hit a matched budget.

The mode that has a working rate parameter cannot be expressed. Proposal §5.5 nominates λ_max for the stochastic form, which would be a genuine rate parameter. But equation (25)'s λ_max and z_0 are not constructor arguments in the specified signature, so the deterministic mode has a parameter that does not work and the stochastic mode has one that cannot be set. Resolving this changes the constructor signature, which is why the encoder has not been written — writing it first would risk rework of the interface (Q11).

8.4  A second, independent block

Q12 records two further under-specifications that a Layer 3 reimplementation would resolve differently. First, the specification does not say which envelope gates the zero crossings — the Hilbert envelope and the rectify-lowpass envelope give different gates, and the difference is visible in the counts. Second, it does not say what parameters the LIF fallback uses above f_lock. Both are subordinate to Q11 in the sense that the constructor signature must be settled first, but neither is implied by it.

The practical consequence of the first gap is already measurable. The recorded span above reproduces the earlier probe's conclusion exactly — 1.039 against 1.04 — but not its absolute counts, which were 7116 falling to 6864 where this run gives 6996 falling to 6732. Neither probe recorded which envelope it used, and that is almost certainly the whole of the difference. The current recorded run declares the Hilbert envelope in its config.

8.5  Output characteristics, as specified

Unipolar, and by a wide margin the densest of the candidates: phase locking at kilohertz rates implies high event counts in exactly the channels where events are cheapest to generate. All information resides in the precise timing of events relative to the carrier, which is what vector strength measures and what the jitter corruption operator is designed to degrade in an interpretable way.

The most interesting predicted outcome in the study. P-03 predicts E5 much the strongest on T2, by a wide margin; not obviously better than E1 on T1, since it adds carrier detail that phone identity does not need; and expensive. That makes it the encoder most likely to be dominated at matched budget despite carrying the most information — an outcome worth stating clearly if it occurs, since it is a concrete illustration of why information content alone is the wrong criterion. E5 is also the encoder that most concerns the dissemination question: if it wins on breadth, the release retains a great deal of speaker information.

9.  E6 — Time-to-first-spike

Status: specified, not implemented. Blocked on Q14 and Q16. The extreme temporal case and the sparsest scheme in the set. The exact inverse of E1: all information in timing, none in event count, which makes the pair (E1, E6) the cleanest available test of whether timing buys anything at matched budget.

9.1  Methodology

The signal is divided into frames of length T_f with hop H. Within each frame each channel emits at most one event, at a latency that decreases with channel energy. Taking the energy in frame m as equation (27), the event time may be set by a direct logarithmic mapping, equation (28), clipped to the frame — or, more principled, by the latency of a LIF neuron under constant current proportional to that energy, equation (29):

E_c[m] = ∫ e_c(t)² dt  over frame m    (27)

t_c[m] = m·H + T_f·( 1 − (log E_c[m] − log E_min)/(log E_max − log E_min) )   (28)

t_c = τ_m · log( I / (I − θ) )   for I > θ    (29)

Channels whose energy falls below E_min emit nothing, which is the mechanism by which the scheme is sparsified. Frame energy is defined in the specification as the sum of squared drive samples in the frame, computed on whatever drive is supplied with no further transformation, so that a test can reproduce it independently.

9.2  Design parameters

| Parameter | Default | Role | Swept? |
| --- | --- | --- | --- |
| e_min | 1e−6 | Energy gate — the declared RATE_PARAM | Intended rate axis — see Q14 |
| frame | 0.025 s | Frame length T_f | Secondary axis |
| hop | 0.010 s | Frame hop H; bounds the budget at N_ch/H events per second | Secondary rate axis (§5.6) |
| tau_m | 0.02 s | Membrane time constant for the lif mode, equation (29) | Secondary axis |
| theta | 1.0 | Threshold for the lif mode, equation (29) | Secondary axis |
| mode | log | log uses equation (28); lif uses equation (29) | Yes |
| E_max | not defined | Normalisation ceiling in equation (28) — defined nowhere | Cannot be set |

9.3  The rate parameter fails, but not structurally

At the declared default the event count is pinned at exactly the ceiling — every channel fires in every frame at every point in the sweep.

|  | 0.25× | 0.5× | 1× | 2× | 4× |
| --- | --- | --- | --- | --- | --- |
| e_min | 2.5e-07 | 5e-07 | 1e-06 | 2e-06 | 4e-06 |
| events | 792 | 792 | 792 | 792 | 792 |

Manifest id e6_rate_parameter_span. Span 1.000×, and 792 is exactly the ceiling N_ch × N_frames. Frame energy on this drive runs min 6.24, median 285.9, max 1852 — so the default e_min sits 6.8 decades below the quietest frame in the signal and gates nothing at all.

This is a default in the wrong place, not a structural defect — and the test suite predicts otherwise. test_G3's own docstring names E6 as the foreseeable casualty of D27, on the grounds that its count is "structurally fixed", and prescribes taking matched budgets from channel count or frame rate instead. That does not describe E6. Swept over the range the energies actually occupy, e_min moves the count across the full dynamic range: 792, 744, 640, 534, 401, 186, 78, 0. The parameter works; the default is six-point-eight decades away from where the signal is. Any base between 205.8 and 462.8 clears the required 4× without an endpoint at extinction. The remedy the docstring prescribes is not needed (Q14).

The fix cannot be made from the implementation side. The operating point lives in the test registry and the signature default lives in the specification, both of which are design-session files, so there is no change available in src/ that turns the test green. Q14 puts two options: retune the absolute value, or make e_min relative to the maximum frame energy, which is scale-free — verified to give identical counts across five decades of drive scale — and which would also define the E_max that equation (28) needs.

9.4  Three further gaps found before writing a line (Q16)

Probing the specification ahead of implementation found three problems, none of which needed the encoder to exist:

| # | Finding | Consequence |
| --- | --- | --- |
| 1 | Equation (28) has no E_max. Not a constructor argument, not defined in the specification or the proposal. | Utterance-max, frame-max and a fixed constant all give different event times; a Layer 3 reimplementation has nothing to choose between them. |
| 2 | Equation (28) is not evaluable at e_min = 0, which is what two of its own tests pass, with mode='log' the default. | log 0 = −inf makes the normalised term inf/inf. E_min is doing two jobs — the emission gate and the normalisation floor — and zero is legal for the first and not the second. |
| 3 | test_T6_1 and test_T6_2 are unsatisfiable by any implementation at hop < frame. | Both pass frame = 25 ms, hop = 10 ms, so windows overlap by 15 ms, and both then treat the window as holding one frame's events when it holds three. |

The third was separated into its two causes by measurement rather than argument, on a prototype that was then discarded:

| hop | Offset clipped to | test_T6_1 duplicate windows | test_T6_2 worst ¦ρ + 1¦ |
| --- | --- | --- | --- |
| 10 ms (default) | frame | 98 of 98 | 1.2301 |
| 10 ms | strictly inside | 98 of 98 | 1.2301 |
| 25 ms (= frame) | frame | 1 of 40 | 0.6000 |
| 25 ms (= frame) | strictly inside | 0 of 40 | 0.0000 |

8 channels, 1 s. The overlap is fatal and no implementation choice touches it. The single remaining failure at hop = frame is the equation (28) boundary — E = E_min maps to an offset of exactly T_f, which lands on m·H + T_f and so falls outside its own half-open window and into the next.

9.5  Output characteristics, as specified

Unipolar, and the sparsest scheme in the set by a wide margin. The event budget is bounded exactly at N_ch / H events per second, which makes E6 uniquely predictable in cost — a genuine practical virtue for a released format, and the reason the hop is a meaningful secondary rate axis in its own right. All information resides in timing; the code carries nothing in event count, which is what makes the (E1, E6) pair the cleanest available test of whether timing buys anything at matched budget.

Predicted behaviour (P-04): competitive on T1 at very low event rates, since a frame-wise spectral snapshot is close to what a conventional filterbank feature vector provides; poor on T3, since temporal resolution is quantised to the frame; poor on T2. Its interest lies at the low-rate end of the Pareto front, where it may dominate everything else.

One part of E6 is fully specified and reachable now: mode='lif' uses equation (29), which involves no E_min or E_max, and test_T6_3 sets hop = frame so no overlap arises. It reproduces its own closed form — I = 4.0, latency 5.7536 ms, comfortably inside the 25 ms frame.

10.  E7 — Spiketrum, and the two reference points

Status: deliberately not implemented (D09). Oliver and Simon approach Wijekoon directly; E7 will not be reimplemented from the published description, to avoid reimplementing a colleague's algorithm badly. Until that conversation has happened the encoder is provisional and is excluded from the critical path.

10.1  E7 — Spiketrum

Spiketrum is a general-purpose spike-coding algorithm developed at Manchester by Alsakkal and Wijekoon, with published FPGA and ASIC implementations and reported application to auditory perception tasks. From the published abstracts it is a sparse decomposition method in the matching-pursuit family: the signal is approximated by iteratively selecting, from a time-frequency dictionary of atoms, the atom best correlated with the current residual, emitting an event identifying that atom and its time, and subtracting its contribution:

(c_k, t_k) = argmax | ⟨ r^(k−1), g_{c,t} ⟩ |    (30)

r^(k) = r^(k−1) − ⟨ r^(k−1), g_{c_k,t_k} ⟩ · g_{c_k,t_k}    (31)

The event train is the sequence of selected atom indices and times, and the number of atoms retained per unit time is a directly controllable rate parameter — which, notably, is the property E5 and E6 are currently blocked on. The reported properties of precisely controllable spike rate, robustness to spike loss, and signal reconstruction all follow from this structure.

Its reconstruction capability is exactly the tension noted for E2, in sharper form: an encoder that optimises reconstruction fidelity is optimising the criterion the study argues is the wrong one, and is by construction the worst case for retained speaker information. That is not a criticism of Spiketrum, which was designed for a different purpose, but it makes the encoder an informative extreme point and a useful upper reference for the privacy axis.

Caveat requiring action, carried from the proposal. The description above is reconstructed from published abstracts and citation records; the Spiketrum papers have not been read in full. Before this encoder is described in a paper or implemented against, the primary sources should be obtained — principally the TETCI article on neuromorphic auditory perception, the TCSI article on the FPGA cochlea, and the evaluation paper on the encoder.

10.2  R1 — Lauscher / Heidelberg reference point

Not a candidate but a fixed reference. The Spiking Heidelberg Digits dataset was produced using an artificial cochlea model with seven hundred output channels, comprising a hydrodynamic basilar membrane model, a transmitter-pool hair cell stage and a bushy cell layer. Including it costs little and buys two things: interoperability, since a substantial body of existing spiking network tooling reads data in that channel format, and a familiar coordinate for readers who know the SHD benchmark. Note that channel count in this study is explicitly a free swept parameter and is not fixed to 700 (D05); whether 700 should nonetheless be a target for the release, for interoperability, is a separate question.

10.3  R2 — Non-spiking upper bound

The essential control, and arguably the single most important row in the results table. Forty mel-scale filterbank features computed over 25 ms windows at 10 ms hop, fed to the same decoder architecture as every spiking condition. This is the standard front end used by Wu and colleagues and by Bittar and Garner, so results are directly comparable to the published literature as well as internally.

Every accuracy should be reported as a gap to this bound. Without it a phone accuracy figure means nothing: the reader cannot tell whether a shortfall reflects the encoding, the decoder, the corpus or the task definition. With it, the central engineering question of the project has a direct numerical answer — what accuracy is given up, at what saving in operations, by representing the audio as events. The headline figure of the paper is a panel of Pareto plots, one per task, with R2 drawn as a horizontal line on each.

11.  Shared machinery: featurisation, corruption, metrics

Three pieces of the harness sit between the encoders and the probe tasks. They are shared deliberately: the proposal is explicit that the conversion from events to decoder input must be identical across encoders or it silently becomes part of what is being compared.

11.1  Featurisation — the encoder/probe interface

Events are converted to a fixed-rate feature vector by an exponential kernel sampled at a fixed frame rate:

φ_c(t) = Σ_k κ(t − t_k),   κ(u) = exp(−u / τ_φ) for u ≥ 0, else 0    (32)

ON and OFF are accumulated separately rather than summed, because cancelling them would discard the distinction a bipolar encoder went to the trouble of making. Encoders declaring a single polarity leave the OFF half at zero, which is correct rather than wasteful: the probe sees the same feature width for every encoder at a given channel count. The implementation accumulates recursively, which is O(N + F) rather than O(N × F), and sorts events into a canonical order before accumulating anything — so the result is bit-identical under any permutation of the input, which is stronger than the tolerance the order-invariance test allows.

| Encoder | Events | Rel. error vs eq (32), scaled | Rel. error, pointwise | Bit-exact under permutation | Polarity folding error |
| --- | --- | --- | --- | --- | --- |
| E1 | 68 | 9.20e-16 | 2.01e-14 | yes | 0.0e+00 |
| E3 | 95 | 1.31e-15 | 2.21e-14 | yes | 1.1e-16 |
| E4 | 30 | 9.39e-16 | 2.03e-14 | yes | 0.0e+00 |

Manifest id featurise_accuracy. Checked against a literal double-sum transcription of equation (32) rather than against its own test. Two relative measures are recorded because they answer different questions: the scaled measure divides by the largest value in the array and is the error a probe would see; the pointwise measure divides by each entry's own value and is dominated by entries decayed to near nothing.

A correction to the record. The notebook quotes 9.4e−16 as the worst relative error. That figure identifies its own normalisation — it is the scaled measure, and E4 gives 9.39e−16 — but it was not the worst of the three: E3 gives 1.31e−15 under the same measure, about 40 per cent larger. The conclusion is unaffected, since agreement is a few units in the last place of the largest feature value either way, but the figure as quoted understates the worst case (Q15).

11.2  Corruption operators

Four operators degrade an event train in controlled ways, used by preliminary experiment P2 and by E5's jitter test. Three are complete and verified: jitter, which perturbs timestamps while preserving count and ordering; delete, which drops events at a given rate; randomise_times, which preserves per-channel counts but destroys timing; and channel_shift, which drops rather than wraps at the edges of the bank. The deliberate design is that different tasks should degrade under different operators — prediction P-07 — so that the battery demonstrably spans the demand space.

One test currently fails on its own precondition rather than on the operator: test_corrupt_delete_retains_expected_fraction guards that the train has more than 500 events, and the fixture yields 275. The operator itself is correct — retention is 0.7382, inside the required window — and the 275 is exact rather than a lost-event bug, identical at 2×, 4×, 8× and 16× oversampling. The guard is doing real statistical work and should not simply be deleted: at 275 events the assertion window is ±1.81 binomial standard deviations and holds for 93.1 per cent of seeds, against ±2.44 and 97.7 per cent at 500 (Q13).

11.3  Reported metrics

| Metric | Definition | Purpose |
| --- | --- | --- |
| Event rate R | N_events / (N_ch · D) | Per-channel rate, equation (34) |
| Total rate Λ | N_events / D | The Pareto x-axis, equation (35) |
| Bit rate B | Λ · (log₂ N_ch + b_t + b_p) | Channel cost, equation (36) |
| Operations | N_events · N_hidden accumulates | Compute cost, equation (37) |
| Information | I(Y ; Ŷ), bits per event and per second | Equations (38)–(39) |
| Vector strength | (1/N)·¦Σ exp(j2πf_c t_k)¦ | E5 diagnostic, equation (26) |
| Temporal information index | (A_temporal − A_count) / (A_ceiling − A_count) | Equation (40); ceiling from R2 |

SpiNNaker energy measurement is out of scope for this study; spike-count and information-per-spike metrics are in scope now (D08).

12.  Recorded results

Every figure quoted in this report that carries a manifest identifier is reproducible from the repository: the manifest names the script, the committed configuration, the commit hash the tree was at, and the seed. The recorder refuses to run unless the script and its config are themselves committed and unmodified, so a recorded commit hash always contains the code that produced the number (D35).

| Result id | Script | Commit | Seed | Date | Output |
| --- | --- | --- | --- | --- | --- |
| front_end_group_delay_residual | measure_group_delay_residual.py | 2017ecff | 0 | 2026-09-05 | 2 files |
| envelope_cutoff_comparison | measure_envelope_cutoff.py | 1e57cece | 0 | 2026-09-05 | 1 file |
| e5_rate_parameter_span | measure_rate_parameter_span.py | 1e57cece | 0 | 2026-09-05 | 1 file |
| e6_rate_parameter_span | measure_rate_parameter_span.py | 1e57cece | 0 | 2026-09-05 | 1 file |
| featurise_accuracy | verify_featurise_accuracy.py | 1e57cece | 0 | 2026-09-05 | 1 file |

Small results are JSON under results/ and committed; bulk arrays go to .npz, which is gitignored, so numbers that reach the paper stay in the repository while large arrays stay local.

Two cautions about what these results are and are not. First, none of them is a task result. No probe has been run, no corpus has been touched, and the TIMIT licence question is still open. Everything recorded so far characterises the encoders and the harness, not their performance on T1, T2 or T3. Second, three seeds are the minimum for anything reported, but every measurement here is on a deterministic stimulus through a deterministic path, where there is nothing for a seed to vary; each config says so explicitly rather than leaving a reader to wonder why one seed was enough.

12.1  Known-answer suite by block

| Block | Covers | Passed | Failed | Blocked on |
| --- | --- | --- | --- | --- |
| F | Front end — filterbank, envelope, group delay | 6 | 0 | Complete |
| T1 | E1 LIF | 3 | 0 | Complete |
| T2 | E2 send-on-delta | 9 | 0 | Complete |
| T3 | E3 temporal contrast | 6 | 0 | Complete |
| T4 | E4 adaptive LIF | 3 | 1 | Q10 |
| T5 | E5 phase-locked | 0 | 5 | Q11, Q12 — encoder not written |
| T6 | E6 time-to-first-spike | 0 | 3 | Q14, Q16 — encoder not written |
| G | Generic, parametrised over all six encoders | 28 | 13 | E5 and E6 stubs |
| corrupt | Corruption operators | 3 | 1 | Q13 |

Totals: 61 passed, 23 failed, 1 skipped. Every failure is attributable to an open question; none indicates a defect in implemented code. Continuous integration runs the same suite in a clean environment on every push and reports the identical failure set.

13.  Open questions, and what each unblocks

Nine questions are open against the design session. They are listed here in the order that clears the most work, which is not the order they were raised. Two pairs should be answered together: Q11 with Q12, because both bear on E5's constructor signature, and Q14 with Q16, because making e_min relative to the maximum frame energy would define the E_max that equation (28) needs.

| Question | Issue | Area | Substance | What answering it unblocks |
| --- | --- | --- | --- | --- |
| Q11 + Q12 | #2, #3 | E5 | E5's declared rate parameter spans 1.04× not 4×, and the mode with a working parameter cannot be expressed; plus two under-specifications | 12 tests — E5's G block and the whole T5 block |
| Q16 + Q14 | #7, #5 | E6 | Equation (28) has no E_max and is not evaluable at e_min = 0; test_T6_1/T6_2 unsatisfiable at hop < frame; e_min default gates nothing | 8 tests — E6's G block and T6 |
| Q10 | #1 | E4 | test_T4_3 asserts a monotonicity the ALIF does not have; onset emphasis peaks near delta_a ≈ 1 | 1 test — but bears on prediction P-01 and on the delta_a sweep range |
| Q13 | #4 | corrupt | Fixture yields 275 events where its own guard requires 500 | 1 test |
| Q15 | #6 | record | Two figures quoted in the record do not reproduce as stated; in both cases the metric was never defined | Nothing — corrects the record |
| Q07 | — | release | ON and OFF as separate channel indices, or as a polarity bit | Nothing — release-format question for Oliver |
| Q09 | — | E3 | test_T3_6 says d[:, 0] is exactly zero; in floating point it usually is | Nothing |

If only one answer comes back, Q10 is the one that matters most. It is worth a single red test and nothing else in implementation terms, but it is the only open item with consequences for a pre-registered prediction. P-01 predicts T1 accuracy rising and T2 falling as adaptation strength increases; if the onset emphasis underneath is non-monotone with a peak near delta_a ≈ 1, a sweep spanning that peak could confirm or contradict P-01 according to which side its points fall.

14.  Pre-registered predictions

These were recorded and dated before any corresponding run, per §7 of the validation protocol, and are held fixed thereafter. A result contradicting one triggers a written investigation before it is used, whatever the outcome. The purpose is not to protect the predictions — several will very likely be wrong, and that is fine and interesting — but to ensure that a surprising result prompts scrutiny rather than a quiet adjustment of the analysis until it stops being surprising.

| # | Prediction | Bears on | Status |
| --- | --- | --- | --- |
| P-01 | E4: T1 accuracy rises and T2 accuracy falls as adaptation strength increases | E4, T1/T2 | open |
| P-02 | E3: strongest on T3; poor on T2, possibly close to useless | E3 | open |
| P-03 | E5: strongest on T2 by a wide margin; likely dominated at matched budget | E5 | open |
| P-04 | E6: dominates the low-rate end of the T1 front; poor on T3 | E6 | open |
| P-05 | E2: strong on T3; its case rests on format symmetry, not peak accuracy | E2 | open |
| P-06 | P1: temporal information index high for T2 and T3, moderate for T1 | Probe P1 | open |
| P-07 | P2: the three tasks degrade under different corruption operators | Probe P2 | open |
| P-08 | Parameters (channel count, spacing, compression) matter more than choice of scheme | Front end | open |

Why this matters more than usual here. The proposal makes specific directional predictions. If the same process both runs the sweeps and writes the analysis, a large number of small choices about parameter ranges, seeds, plot limits and which runs to repeat will tend to favour those predictions. No dishonesty is required for this; it is the ordinary mechanism by which analysis drifts toward its hypothesis, and an assistant that has read the proposal is if anything more susceptible than a person, not less. Parameter grids, seed counts and sweep ranges are therefore fixed in committed configuration before the sweep runs, and extending a range after seeing results is legitimate only if declared as such in the paper.